Weight is measurement by means of a balance. There are different types of balance, types for   rough weights and others for sensitive weights.

Rules for the use and care of the balance

1-the balance should be located in a well –lightened place ,as far as possible from vibration ,dust, moisture and corrosive vapor.

2-the balance should be kept clean at all times.

3-watch glass should be used for weighing of solids, stoppered bottles used for corrosive substances and liquids

4-check to see if the balance is adjusted including  zeroing of it ,also check the adjustment before each weighing.

Source of error in weighing :

1-failure to adjust the scale .

2-currents of air.

Systems of measuring weights

1-Imperial system which is mainly used in UK and  USA, example (grain ,ounce, pound, and drachm).

2-metric system example(  gram , kilogram, milligram, microgram ).

The relation between metric and imperial system

- 1 grain =0.065 g

- 1 gram =15.432 grain

- 1 grain = 65 mg

- 1 drachmae = 60gr ≈ 4 g

- 1 ounce ≈ 30 g

- 1 kg = 2.2 pound

General notes

- A clean spatula may be put  into a bottle to remove solids for weighing .

- Do not contaminate  reagents by using dirty equipment's  or improper techniques.

- Bottle should be kept clean and restopper each reagent bottle with it's own stopper .

- Never place materials to be weighed directly on the pans .

- Accuracy requires both understanding and care .

- Most countries are following  the metric system of weight  and  measure.