Network traffic flow fundamentals

Coarse model of network dynamics

Fundamental relation between network outflow (rate at which trip end) and accumulation.

Demand and performance degradation

Important characteristic of traffic networks:

􀀁 Network production degenerates as number of vehicles surpasses the

critical number of vehicles in the network.

􀀁 Expressed by the Macroscopic (or Network) Fundamental Diagram.

First model of traffic congestion

Fundamental diagram

􀁎 Relation between traffic density and traffic speed: u = U(k)

􀁎 Underlying behavioural principles? (density = 1/average distance)

Dynamic properties

Traffic congestion at bottleneck (on-ramp)

􀁎 Consider bottleneck due to on-ramp.

􀁎 Resulting capacity (capacity – ramp flow) is lower than demand.

􀁎 Queue occurs upstream of bottleneck and moves upstream as long as upstream demand > flow in queue (shockwave theory).

Shockwave theory

􀁎 Predicting queue dynamics (queuing models, shockwave theory)

􀁎 Predicts dynamics of congestion using FD

􀁎 Flow in queue = C – q(on-ramp)

􀁎 Shock speed determined by:

Dynamic features of road congestion

Capacity funnel, instability, wide moving jams

􀁎 Capacity funnel (relaxation) and capacity drop.

􀁎 Self-organisation of wide moving jams.

Capacity drop

Two capacities

Instability and wide moving jams

􀁎 Wide moving jams can exist for hours and travel past bottlenecks.

􀁎 Density in wide moving jam is very high (jam-density) and speed is low.

Emergence and dynamics of start-stop waves

Pedestrian flow congestion

Start-stop waves in pedestrian flow

Example of Jamarat bridge shows self-organized stop-go waves in pedestrian traffic flows.

Modeling challenge

􀁎 Traffic flow is a result of human decision making and multi-actor interactions at different behavioral levels (driving, route choice,

departure time choice, etc.).

􀁎 Characteristics behavior (inter- and intra-driver heterogeneity)

􀀁 Large diversity between driver and vehicle characteristics.

􀀁 Intra-driver diversity due to multitude of influencing factors, e.g.

prevailing situation, context, external conditions, mood, emotions.

􀁎 The traffic flow theory does not exist (and will probably never

exist): this is not Newtonian Physics or thermodynamics.

􀁎 Challenge is to develop theories and models that represent reality

sufficiently accurate for the application at hand.

Network Traffic Modeling

􀁎 Traffic conditions on the road are end result of many decisions made by the traveler at different decision making levels

􀁎 Depending on type of application different levels are in- or excluded in model

􀁎 Focus on driving behavior and flow operations

Model components and processes

Modeling approaches

Microscopic and macroscopic approaches:

􀁎 Two dimensions:

􀀁 Representation of traffic.

􀀁 Behavioral rules, flow characteristics.

Overview of flow variables

􀁎 Microscopic characteristics

􀀁 Arrival processes

􀀁 Headway models / headway distribution models

􀀁 Critical gap distributions

􀁎 Macroscopic characteristics

􀀁 Fundamental diagram

􀀁 Shockwaves and non-equilibrium flow properties

Traffic Flow Variables

􀁎 Positions xi(t) along roadway of vehicle i at  time t

􀁎 All microscopic and macroscopic characteristics

can be determined from trajectories!

􀁎 In reality, trajectory information is rarely

available.

􀁎 Nevertheless, trajectories are the most important unit of analysis is traffic flow theory.

Vehicle trajectories (2)

􀁎 Which are trajectories?

􀁎 Vehicle trajectories determined using remote sensing data (from helicopter) at site Everdingen near Utrecht.

􀁎 Dots show vehicle position per 2.5 s

􀁎 Unique dataset.

Empirical vehicle trajectories

Time headways

􀁎 Time headway hi: passage time difference rear bumper vehicle i-1 and i at crosssection x (easy to measure).

Time headways (3)

􀁎 Headways are local variables (collected at cross-section x0).

􀁎 Mean headway for certain period T of the n vehicles that have passed x0.

Distance headways

􀁎 Distance headway si: difference between positions vehicle i-1 and i at time t (difficult to measure!).

􀁎 Gross distance headway and net distance headway.

Distance headways (2)

􀁎 Distance headway: instantaneous microscopic variable.

􀁎 Space-mean distance headway of m vehicles at time instant t for roadway of length X.

Macroscopic flow variables

􀁎 Traffic intensity q (local variable)

􀀁 Vehicle number passing cross-section x0 per unit time (hour,

15 min, 5 min).

􀀁 If n vehicles pass during T, q is defined by:

􀁎 Traffic density k (instantaneous variable)

􀀁 Vehicle number present per unit roadway length (1 km, 1 m) at

instant t

􀀁 If m vehicles present on X, k is defined by

Mean speeds

􀁎 Local mean speed / time mean speed

􀀁 speeds vi of vehicles passing a cross-section x during period T

􀁎 Instantaneous / space mean speed

􀀁 speed vj of vehicles present at road section at given moment t

􀁎 We can show that under special  circumstances we can compute space-mean speeds from local measurements

Stationary flow conditions

Homogeneous flow conditions