Traffic Flow Theory and Simulation

Lateral driving behaviour

Three phase traffic flow theory

Capacity drop

- differentiate between courtesy, mandatory and desired lane changes.

- explain gap acceptance theory and comment on measuring critical gaps.

-comment on the principles of the Mobil lane change model (equations will be given).

- comment on lane selection theories.

- comment an analyse lane distributions.

- comment on merging.

- describe the principle of relaxation.

Driving tasks

GAP ACCEPTANCE THEORY

Gap acceptance -concept

Gap acceptance for overtaking

Overtaking in time and space

Gap in time is not completely usable for overtaking

- nett gap starts at t4, when oncomer meets overtaker

- overtaker has to be back on right lane beforet3to prevent a collision

Critical gap distribution

- One measures gap-accepance. Comparing the distribution of the accepted gaps to the critical gap distribution, one finds:A) The critical gaps are longer than the measured gapsB) The measured gaps are shorter than the critical gapsC) If one does enough measurements, they are equal.

Application using the critical gap

Model for ‘one driver at a time’ manoeuvres (e.g. a car crossing an intersection: single queue/server system)

Simple model gap acceptance process for ‘fixed’ critical gap Draw critical gap ci from distribution F(c)

1. Draw offered gap gj from offered gap distribution.

2. If gj >  ci, then gap is accepted; go to 1.

3. If gi <  cj, then gap is rejected; go to 2.

4. If gi <  cj, then gap is rejected; go to 2.

MOTORWAY LANE CHANGING

3 types of lane changing:

- Mandatory

- Desired

- Courtesy

Mandatory lane changes

LANE CHANGE MODELS

Models & observations

-Many lane changing models

-Problem in observation: no reasoning, only events

-Events: willingness x probability

-All models are wrong(but some are more wrong than others)

-Calibration?

Model by Helbing, Treiber, Kesting.

Based on utility.

Minimising Overall Braking Induced by Lane changes

Consider accelerations of 3 vehicles

Mobil: decision

How to calibrate

-Number of lane changes?

-Lane distributions?

-Travel times?

Mandatory lane changes

-We do not know

-Engineers find a solution: Adapt the model!

-Usually: change model parameters until it looks OK

Problems of simulation programs

Cars stuck: possible solutions

-Stop cars until gap is there.

-Remove cars.

-Other direction (in case of wrong descision).

-Cars stuck at the on-rampwww.traffic-simulation.de.

-Low-speed merging creating congestion.

Implications of lateral behaviour

- Lane changes are important to for traffic flows:

..Lane distribution.

..Disturbances.

- But it is very difficult to model them correctly.

… differentiate between courtesy, mandatory and desired lane changes.

… explain gap acceptance theory and comment on measuring critical gaps.

Comment on the principles of the Mobil lane change model (equations will be given)

Dependencies

Parallelograms / Rhomboid

Setup of analysis

Separate densities in origin lane and destination lane

Future conditions not included (drivers anticipate).

Lane changes are induced by lane changes from the target lane (place swapping), which occur more frequently with higher target lane density.

Separation origin lane density and target lane density artificial.

Daganzo’s theory of slugs and rabbits…

Flow distribution

LANE SELECTION THEORIES

Slugs and Rabbits (Daganzo, 2002)

Slugs: lower free speedright lane(s)

Rabbits higher free speed left lane if that is faster than the right lane(s)

Two different regimes

Slugs and rabbits –start of congestion

-Rabbits and slugs enter on-ramp

-Rabbits will not merge immediately into left lane

Consequences of merging

Oversaturation on left lane after lane change.

Start congestion on left lane, downstream of ramp

Impact and consequences

-Capacity drop

-Congestion start downstream of merge (“boomerang”)

-Lane distribution uneven

-All seen in practice!

Observed phenomena in traffic flow

Lane Distribution

Influence of speed limit

Merging

-Helicopter observations.

-Gap acceptance theories are wrong.

-Current research => gap selection theory?

-Relaxation present.

Available gaps

Mobil: safety criterion