Traffic Flow Theory and Simulation

Shockwave Theory

Introduction to shockwave analysis:

• Introduce application of fundamental diagram to shockwave analysis with aim to understand importance of field location.

• Shockwave analysis:

• Vehicles are conserved

• Traffic acts according to the fundamental diagram (q = Q(k))

• Predicts how inhomogeneous conditions change over time.

• FOSIM demonstration

• Example 3 -> 2 lane drop and emerging shockwaves

(road works, incident, etc.).

FOSIM example

• Extremely short introduction to FOSIM

• Build a simple network (8 km road with road works at

x = 5 km to 6 km)

• Implement traffic demand

• Assume 10% trucks

• Suppose that upstream traffic flow > capacity of bottleneck

• What will happen?

• Why does congestion occur

• Macroscopically?

• Microscopically?

Microscopic description

• Congestion at a bottleneck

• Simplest is to compare the system to a (sort of) queuing system

• Drivers arrive at a certain rate (demand) at specific time intervals

• The n ‘servers’ needs a minimum amount of time to process the drivers (each lane is a server)

• Service time T is a driver-specific (random) variable depending on weather conditions, road and ambient conditions, etc.

(= minimum time headway of a driver)

• Note that service time is directly related to car-following behavior

• When another driver arrives when the server is still busy, he / she has to wait a certain amount of time

• Waiting time accumulates - > queuing occurs when the server is still busy, he / she has to wait a certain amount of time

•

Macroscopic description Waiting time accumulates ->

• Compare traffic flow to a fluidic (or better: granular) flow though a narrow bottleneck (hour-glass, funnel)

• If traffic demand at a certain location is larger than the supply (capacity) congestion will occur

• Capacity is determined by number of lanes, weather conditions, driver behavior, etc.

• Excess demand is stored on the motorway to be served in the next time period

• Remainder: focus on macroscopic description.

• Where does congestion first occur?

• Which traffic conditions (traffic phases) are encountered?

• Where are these conditions encountered?

Definition of a shockwave

• Consider over-saturated bottleneck

• Traffic conditions change over space and time

• Boundaries between traffic regions are referred to as shockwaves.

• Shockwave can be very mild (e.g. platoon of high-speed vehicles catching up to a platoon of slower driver vehicles).

• Very significant shockwave (e.g. free flowing vehicles approaching queue

of stopped vehicles).

• Shockwave is thus a boundary in the space-time domain that demarks a discontinuity in flow-density conditions.

• Example: growing / dissolving queue at a bottleneck.

Fundamental diagrams

• What can we say about the FD at the different locations?

• Some ‘standard’ numbers (for Dutch motorways)

• Capacity point ! n * 2200 pce/h/lane

• Critical density ! n * 25 pce/km/lane

• Jam-density ! n * 2200 pce/km/lane

• Critical speed ! 85 km/h

• Free speed: to be determined from speed limit

• pce = ‘person car equivalent’

• Exact numbers depend on specific characteristics of considered

location (traffic composition, road conditions, etc.)

Shockwave equations

• Assume that flow-density relation Q(k) is known for all location x on the road (i.e. different inside and outside bottleneck!)

• Consider queue due to downstream bottleneck

• Consider conditions in the queue

• Flow q2 = Cb-n (capacity downstream bottleneck)

• Speed, density follow from q2, i.e. u2 = U(q2) and k2 = q2/u2

• Farther upstream of queue, we have conditions (k1,u1,q1)

• Speed u1 > u2 " upstream vehicles will catch up with vehicles in

queue.

• Remarks:

• If k2 > k1 sign shockwave speed negative if q1 > q2 (backward forming shockwave).

• If k2 > k1 sign shockwave speed positive if q1 < q2 (forward recovery shockwave).

• If k2 > k1 sign shockwave speed zero if q1 = q2 (backward stationary).

• Classification of shockwaves

• Shockwave theory is applicable when

• Q(k) is known for all location x

• Initial conditions are known

• Boundary conditions (at x1 AND x2) are known

• Shockwaves occur when

• Spatial / temporal discontinuities in speed-flow curve

(expressed Q(k,x)), e.g. recurrent bottleneck

• Spatial discontinuities in initial conditions

• Temporal discontinuities in boundary conditions at x1 (or x2)

Applications of shockwave theory

-Temporary over-saturation.

-Traffic lights.

• Temporary over-saturation of a bottleneck.

• Traffic demand (upstream).

Three simple steps to applying shockwave

Three simple steps to applying shockwave theory:

1. Determine the Q(k) curve for all locations x

2. Determine the following ‘external conditions’:

• initial states (t = t0)

• ‘boundary’ states (inflow, outflow restrictions, moving

bottleneck). present in the x-t plane and the q-k plane

3. Determine the boundaries between the states (=shockwaves) and determine their dynamics

4. Check for any ommisions you may have made (are regions with different states separated by a shockwave).

Shockwaves at Bottleneck

Shockwaves signalized intersections

Shockwave classification

1. Frontal stationary: head of a queue in case of stationary / temporary bottleneck.

2. Forward forming: moving bottleneck (slow vehicle moving in direction of the flow given limited passing opportunities).

3. Backward recovery: dissolving queue in case of stationary or temporary. bottleneck (demand l.t. supply); forming or dissolving queue for moving  bottleneck.

1. Forward recovery: removal of temporary bottleneck (e.g. clearance of incident, opening of bridge, signalized intersection)

2. Backward forming: forming queue in case of stationary, temporary, or moving bottleneck* (demand g.t. supply);

3. Rear stationary: tail of queue in case recurrent congestion

when demand is approximately equal to the supply.

Flow into shockwave