Traffic Operation

1. Road users Vehicle and Roadway Characteristics

To begin to understand the functional and operational aspects of traffic on streets and highways it is important to understand how the various elements of a traffic system interact. Further, the characteristics of traffic streams are heavily influenced by the characteristics and limitations of each of these elements. There are five critical components that interact in a traffic system:

1. Road users-drivers, pedestrians, bicyclists, and

2. Vehicles-private and commercial

3.Streets and highways

4. Traffic control devices

5. The general environment

1.1 Road Users

Human beings are complex and have a wide range of characteristics that can and do influence the driving task. In a system where the driver is in complete control of vehicle operations, good traffic engineering requires a keen understanding of driver characteristics. Much of the task of traffic engineers is to find ways to provide drivers with information in a clear, effective manner that induces safe and proper responses.

The two driver characteristics of utmost importance are visual acuity factors and the reaction process.

The two overlap, in that reaction requires the use of vision for most driving cues. Understanding how information is received and processed is a key element in the design of roadways and controls.

1.1.1 Visual Characteristics of Drivers

Visual Factors in Driving

Many of the other factors listed in Table 1 reflect the dynamic nature of the driving task and the fact that most objects to be viewed by drivers are in relative motion with respect to the driver's eyes.

Table (1): Visual Factors in the Driving Task

1.1.2 Fields of Vision

Figure 1 illustrates three distinct fields of vision, each of which is important to the driving task:

1. Acute or clear vision cone-3" to 10" around the line of sight; legend can be read only within this

narrow field of vision.

2. Fairly clear vision cone-10" to 12" around the line of sight; color and shape can be identified in this field.

3. Peripheral vision-This field may extend up to 90" to the right and left of the centerline of the pupil, and up to 60" above and 70" below the line of sight. Stationary objects are generally not seen in the peripheral vision field, but the movement of objects through this field is detected.

Figure 1: Illustration of Fields of Vision.

1.1.3 Perception-Reaction Time

The second critical driver characteristic is perception reaction time (PRT). During perception and reaction, there are four distinct processes that the driver must perform:

1. Detection. In this phase, an object or condition of concern enters the driver’s field of vision, and the driver becomes consciously aware that something requiring a response is present.

2. Identification. In this phase, the driver acquires sufficient information concerning the object or condition to allow the consideration of an appropriate response.

3. Decision. Once identification of the object or condition is sufficiently completed, the driver must analyze the information and make a decision about how to respond.

4. Response. After a decision has been reached, the response is now physically implemented by the driver.

The total amount of time that this process takes is called the perception-reaction time (PRT). In some of the literature, the four phases are referred to as perception, identification, emotion, and volition, leading to the term “PIEV time.” This text will use PRT, but the reader should understand that this is equivalent to PIEV time.

1.1.4 Design Values

Like all human characteristics, perception-reaction times vary widely amongst drivers, as do a variety of other factors, including the type and complexity of the event perceived and the environmental conditions at the time of the response.

Nevertheless, design values for various applications must be selected. The American Association of State Highway and Transportation Officials (AASHTO) mandates the use of 2.5 seconds for most computations involving braking reactions, based upon a number of research studies. This value is believed to be approximately a 90th percentile criterion (i.e., 90% of all drivers will have a PRT as fast or faster than 2.5 s).

For signal timing purposes, the Institute of Transportation Engineers  recommends a PRT time of 1.0 second. Because of the simplicity of the response and the preconditioning of drivers to respond to signals, the PRT time is significantly less than that for a braking response on an open highway. While this is a lower value, it still represents an approximately 85th percentile for the particular situation of responding to a traffic signal.

1.1.5 Expectancy

The concept of expectancy is important to the driving task and has a significant impact on the perception reaction process and PRT. Simply put, drivers will react more quickly to situations they expect to encounter as opposed to those that they do not expect to encounter. There are three different types of expectancies:

1.  Continuity. Experiences of the immediate past are generally expected to continue. Drivers do

not, for example, expect the vehicle they are following to suddenly slow down.

2. Event. Things that have not happened previously will not happen. If no vehicles have been observed entering the roadway from a small driveway over a reasonable period of time, then the driver will assume that none will enter now.

3.  Temporal. When events are cyclic, such as a traffic signal, the longer a given state is observed, drivers will assume that it is more likely that a change will occur.

The impact of expectancy on PRT is illustrated in Figure 2.

Figure 2: Comparison of Perception-Reaction Times Between Expected and Unexpected Events.

1.1.6 Other Factors Affecting PRT

In general. PRTs increase with a number of factors, including :

(1) age,

(2) fatigue,

(3) complexity of reaction, and

(4)  presence of alcohol and/or drugs in the driver’s system. While these trends are well documented, they are generally accounted for in recommended design values, with the exception of the impact of alcohol and drugs.

1.1.7 Reaction Distance

The most critical impact of perception-reaction time is the distance the vehicle travels while the driver goes through the process. In the example of a simple braking reaction, the PRT begins when the driver first becomes aware of an event or object in his or her field of vision and ends when his or her foot is applied to the brake.

During this time, the vehicle continues along its original course at its initial speed. Only after the foot is applied to the brake pedal does the vehicle begin to slow down in response to the stimulus.

The reaction distance is simply the PRT multiplied by the initial speed of the vehicle. As speed is generally in units of mi/h and PRT is in units of seconds, it is convenient to convert speeds to ft/s for use:

Thus, the reaction distance may be computed as:

d = 1.47St

where:

d = reaction distance, ft

t = reaction time, s

S= initial speed of vehicle, mi/h

2. Pedestrian Characteristics

One of the most critical safety problems in any highway and street system involves the interactions of vehicles and pedestrians. A substantial number of traffic accidents and fatalities involve pedestrians. This is not surprising, as in any contact between a pedestrian and a vehicle, the pedestrian is at a significant disadvantage.

Virtually all of the interactions between pedestrians and vehicles occur as pedestrians cross the street at intersections and at mid-block locations. At signalized intersections, safe accommodation of pedestrian crossings is as critical as vehicle requirements in establishing an appropriate timing pattern. Pedestrian walking speed in crosswalks is the most important factor in the consideration of pedestrians in signal timing.

At unsignalized crossing locations, gap-acceptance behaviour of pedestrians is another important consideration.

“Gap acceptance” refers to the clear time intervals between vehicles encroaching on the crossing path and the behaviour of pedestrians in “accepting” them to cross through.

2.1 Walking Speeds

Table 2 shows 50th percentile walking speeds for pedestrians of various ages. It should be noted that these speeds were measured as part of a controlled experiment  and not specifically at intersection or mid-block crosswalks. Nevertheless, the results are interesting. The standard walking speed used in timing signals is 4.0 ft/s, with 3.5 ft/s recommended where older pedestrians are predominant. Most studies indicate that these standards are reasonable and will accommodate 85% of the pedestrian population.

Table 2: 50th Percentile Walking Speeds for Pedestrians of Various Ages.

2.2 Gap Acceptance

When a pedestrian crosses at an uncontrolled (either by signals, STOP, or YIELD signs) location, either at an intersection or at a mid-block location, the pedestrian must select an appropriate “gap” in the traffic stream through which to cross. The “gap” in traffic is measured as the time lag between two vehicles in any lane encroaching on the pedestrian’s crossing path. As the pedestrian waits to cross, he or she views gaps and decides whether to “accept” or “reject” the gap for a safe crossing. Some studies have used a gap defined as the distance between the pedestrian and the approaching vehicle at the time the pedestrian begins his or her crossing.

An early study  using the latter approach resulted in an 85th percentile gap of approximately 125 ft.

Gap acceptance behaviour, however, is quite complex and varies with a number of other factors, including the speed of approaching vehicles, the width of the street, the frequency distribution of gaps in the traffic stream, waiting time, and others.

3. Vehicles Characteristics

In general, motor vehicles are classified by AASHTO 151 into four main categories:

- Passenger curs-all passenger cars, SUVs,

- minivans, vans, and pickup trucks.

- Buses-intercity motor coaches, transit buses,

- school buses, and articulated buses

- Trucks-single-unit trucks, tractor-trailer, and

- tractor-semi-trailer combination vehicles

- Recreational vehicles-motor homes, cars with various types of trailers (boat, campers, motorcycles, etc.).

There are a number of critical vehicle properties that must be accounted for in the design of roadways and traffic controls. These include:

- Braking and deceleration

- Acceleration

- Low-speed turning characteristics

- High-speed turning characteristics

3.1 Turning Characteristics of Vehicles

There are two conditions under which vehicles must make turns:

Low-speed turns (≤ 10 mi/h)

High-speed turns (> 10 mi/h)

Low-speed turns are limited by the characteristics of the vehicle, as the minimum radius allowed by the vehicle’s

steering mechanism can be supported at such speeds. High-speed turns are limited by the dynamics of side friction between the roadway and the tires, and by the superelevation (cross-slope) of the roadway.

3.2 Low Speed Turns

AASHTO specifies minimum design radii for each of the design vehicles, based on the centerline turning radius and minimum inside turning radius of each vehicle. While the actual turning radius of a vehicle is controlled by the front wheels, rear wheels do not follow the same path. They “off-track” as they are dragged through the turning movement.

Figure 3: Low-Speed Turning Template for WB-40 Combination Vehicles (Used with permission of American Association of State Highway and Transportation Officials, A Policy on Geometric Design of Highways and Streets.

Note that the minimum turning radius is defined by the track of the front outside wheel. The combination vehicle, however, demonstrates considerable “off-tracking’’ of the rear inside wheel, effectively widening the width of the “lane” occupied by the vehicle as it turns. The path of the inside rear wheel is not circular, and has a variable radius. Turning templates provide illustrations of the many different dimensions involved in a low-speed turn.

In designing for low-speed turns, the minimum design turning radius is the minimum centerline radius plus one-half of the width of the front of the vehicle.

Minimum design turning radii range from 24.0 ft for a passenger car to a high of 60.0 ft for the WB-109D double tractor-trailer combination vehicle. Depending upon the specific design vehicle, the minimum inside curb radius is generally considerably smaller than the minimum design turning radius, reflecting the variable radius of the rear-inside wheel’s track. In designing intersections, off-tracking characteristics of the design vehicle should be considered when determining how far from travel lanes to locate (or cut back) the curb. In a good design, the outside wheel of the turning design vehicle should be able to negotiate its path without “spilling over” into adjacent lanes as the turn is negotiated.

This requires that the curb setback must accommodate the maximum off-tracking of the design vehicle.

3.3 High-Speed Turns

When involved in a high-speed turn on a highway curve, centripetal forces of momentum are exerted on the vehicle to continue in a straight path. To hold the curve, these forces are opposed by side friction and superelevation. Superelevation is the cross-slope of the roadway, always with the lower edge in the direction of the curve.

The sloped roadway provides an element of horizontal support for the vehicle. Side-friction forces represent the resistance to sliding provided across the plane of the surface between the vehicle’s tires and the roadway.

From the basic laws of physics, the relationship governing vehicle operation on a curved roadway is:

where:

e = superelevation rate, %

f = coefficient of side friction

S = speed of the vehicle, ft/s

R = radius of curvature, ft

g = acceleration rate due to gravity, 32.2 ft/s2

Coefficients of side friction for design are based upon wet roadway conditions. They vary with speed and are shown in Table 3.

Theoretically, a road can be banked to fully oppose centripetal force without using side friction at all. This is, of course, generally not done, as vehicles travel at a range of speeds and the superelevation rate required in many cases would be excessive. High-speed turns on a flat pavement may be fully supported by side friction as well, but this  generally limits the radius of curvature or speed at which the curve may be safely traversed.

Table 3: Coefficient of Side Friction (f) for Wet Pavements at Various Speeds.

3.4 Braking Characteristics

Another critical characteristic of vehicles is their ability to stop (or decelerate) once the brakes have been engaged.

Again, basic physics relationships are used. The distance traveled during a stop is the average speed during the stop multiplied by the time taken to stop, or:

where:

db  = braking distance, ft.

S    = initial speed, ft/s.

a    = deceleration rate, ft/s2.

When the effects of grade are considered, and where a braking cycle leading to a reduced speed other than “0” are considered, the equation becomes:

db=Si2-Sf230F∓0.01G

where:

G = grade, %.

Si= initial speed.

Sf= final speed.

When there is an upgrade, a + is used; a -  is used for downgrades. This results in shorter braking distances on upgrades, where gravity helps deceleration, and longer braking distances on downgrades, where gravity is causing acceleration.

3.5 Acceleration Characteristics

where:

da = acceleration distance, ft.

S = speed at the end of acceleration (from a stop), mi/h.

a = acceleration rate, ft/s2.

3.6 Sight Distance

The safe and efficient operation of vehicles on the road depends very much on the visibility of the road ahead of the driver. Thus the geometric design of the road should be done such that any obstruction on the road length could be visible to the driver from some distance ahead . This distance is said to be the sight distance.

3.6.1 Types of Sight Distance

- Stopping sight distance (SSD) or the absolute minimum sight distance.

- Intermediate sight distance (ISD) is defined as twice SSD.

- Overtaking sight distance (OSD) for safe overtaking operation.

- Head light sight distance is the distance visible to a driver during night driving under the illumination of head lights.

- Safe sight distance to enter into an intersection.

The most important consideration in all these is that at all times the driver traveling at the design speed of the highway must have sufficient carriageway distance within his line of vision to allow him to stop his vehicle before colliding with a slowly moving or stationary object appearing suddenly in his own traffic lane.

The computation of sight distance depends on:

1. Reaction time of the driver.

2. Speed of the vehicle.

3. Efficiency of brakes.

4. Frictional resistance between the tyre and the road.

5. Gradient of the road.

3.6.2 Stopping sight distance

Stopping sight distance (SSD) is the minimum sight distance available on a highway at any spot having sufficient length to enable the driver to stop a vehicle traveling at design speed, safely without collision with any other obstruction.

3.6.3 Overtaking Sight Distance

The overtaking sight distance is the minimum distance open to the vision of the driver of a vehicle intending to overtake the slow vehicle ahead safely against the trace in the opposite direction. The overtaking sight distance or passing sight distance is measured along the center line of the road over which a driver with his eye level 1.2 m above the road surface can see the top of an object 1.2 m above the road surface.

The factors that affect the OSD are:

- Velocities of the overtaking vehicle, overtaken vehicle and of the vehicle coming in the opposite direction.

- Spacing between vehicles, which in-turn depends on the speed

-  Skill and reaction time of the driver

-  Rate of acceleration of overtaking vehicle

-  Gradient of the road

Figure 4: Time-Space Diagram: Illustration of Overtaking Sight Distance.

The dynamics of the overtaking operation is given in the figure which is a time-space diagram. The x-axis denotes the time and y-axis shows the distance traveled by the vehicles. The trajectory of the slow moving vehicle (B) is shown as a straight line which indicates that it is traveling at a constant speed. A fast moving vehicle (A) is traveling behind the vehicle B. The trajectory of the vehicle is shown initially with a steeper slope.

The dotted line indicates the path of the vehicle A if B was absent. The vehicle A slows down to follow the vehicle B as shown in the Figure 3 with same slope from t0 to t1. Then it overtakes the vehicle B and occupies the left lane at time t3. The time duration T = t3 - t1 is the actual duration of the overtaking operation. The snapshots of the road at time t0; t1, and t3 are shown on the left side of the figure. From the Figure 4, the overtaking sight distance consists of three parts.

-  d1 the distance traveled by overtaking vehicle A during the reaction time t = t1 - t0

-  d2 the distance traveled by the vehicle during the actual overtaking operation T = t3 - t1

-  d3 is the distance traveled by on-coming vehicle C during the overtaking operation (T).

Therefore:

OSD = d1 + d2 + d3

It is assumed that the vehicle A is forced to reduce its speed to vb, the speed of the slow moving vehicle B and travels behind it during the reaction time t of the driver. So d1 is given by:

d1 = vbt

Then the vehicle A starts to accelerate, shifts the lane, overtake and shift back to the original lane. The vehicle A maintains the spacing s before and after overtaking. The spacing s in m is given by:

s = 0.7vb + 6

Let T be the duration of actual overtaking. The distance traveled by B during the overtaking operation is 2s+vbT. Also, during this time, vehicle A accelerated from initial velocity vb and overtaking is completed while reaching final velocity v. Hence the distance traveled is given by:

The distance traveled by the vehicle C moving at design speed v m=sec during overtaking operation is given by:

d3 = v.T

The  overtaking sight distance is:

where vb is the velocity of the slow moving vehicle in m=sec2, t the reaction time of the driver in sec, s is the spacing between the two vehicle in m given by equation;

s = 0.7vb + 6

and a is the overtaking vehicles acceleration in m/sec2. In case the speed of the overtaken vehicle is not given, it can be assumed that it moves 16 km/h slower the design speed.

The acceleration values of the fast vehicle depends on its speed and given in Table 4. Note that:

- On divided highways, d3 need not be considered

- On divided highways with four or more lanes, IRC suggests that it is not necessary to provide the OSD, but only SSD is sufficient.

Table 4: Maximum Overtaking Acceleration at Different Speeds.

3.6.4 Overtaking Zones

Overtaking zones are provided when OSD cannot be provided throughout the length of the highway. These are zones dedicated for overtaking operation, marked with wide roads. The desirable length of overtaking zones is 5 time OSD and the minimum is three times OSD as shown in Figure 5:

Figure 5: Overtaking Zones.

3.6.5 Sight Distance at Intersections

At intersections where two or more roads meet, visibility should be provided for the drivers approaching the intersection from either sides. They should be able to perceive a hazard and stop the vehicle if required. Stopping sight distance for each road can be computed from the design speed. The sight distance should be provided such that the drivers on either side should be able to see each other. This is illustrated in the Figure 6.

Figure 6: Sight Distance at Intersections.

Design of sight distance at intersections may be used on three possible conditions:

- Enabling approaching vehicle to change the speed.

- Enabling approaching vehicle to stop.

- Enabling stopped vehicle to cross a main road.