Traffic Engineering

Volume Studies and Characteristics

Why Traffic Volume Studies

The starting point for most traffic engineering is the current state of facilities and traffic along with a prediction or anticipation of future demand. The former requires that a wide variety of data and information be assembled that adequately describe the current status of systems, facilities, and traffic.

Traffic engineers collect data for many reasons and applications:

1. Managing the physical system. Physical inventories of physical system elements are always needed. These include traffic control devices, lighting fixtures, and roadways. These inventories help assess which items need to be replaced

or repaired, and on what anticipated schedule.

2. Investigating trends over time. Traffic engineers need trend data to help forecast future transportation

needs. Parametric studies on traffic volumes, speeds, and densities help quantify current demand

and assess operating quality of traffic facilities. Accident studies reveal locations with problems that must be addressed and mitigated. Establishing trends allows the traffic engineer to make needed improvements before an obvious deficiency in the system manifests itself.

3. Understanding the needs and choices of the public and industry. The traffic engineer must have a good measure of how and why people travel for planning and development purposes. Studies of how travelers make mode choices, time of trip decisions, and other judgments is critical to understanding the nature of travel demand. Studies of parking and goods delivery characteristics help plan facilities to effectively handle these demands.

4. Calibrating basic relationships or parameters. Fundamental measures, such as perception reaction time, discharge headways at a signalized intersection, headway and spacing relationships on freeways and other uninterrupted flow facilities, and other key parameters and relationships must be properly quantified and calibrated to existing conditions. Such measures are incorporated into a variety of predictive and assessment models on which much of traffic engineering is based.

5. Assessing the effectiveness of improvements. When improvements of any kind are implemented, follow-up studies are needed to confirm their effectiveness, and to allow for adjustments if all objectives are not fully met.

6. Assessing potential impacts. An essential part of traffic engineering is the ability to predict and analyze projected traffic impacts of new developments and to provide traffic input to air pollution models.

7. Evaluating facility or system performance. All traffic facilities and systems must be periodically studied to determine whether they are delivering the intended quantity and quality of access and/or mobility service to the public.

Volume Characteristics

The most fundamental measurement in traffic engineering is counting-counting vehicles, passengers, and/or people. Various automated and manual counting techniques are used to produce estimates of:

-Volume

-Rate of flow

-Demand

-Capacity

Volume is the number of vehicles (or persons) passing a point during a specified time period, which is usually one hour, but need not be.

Rate of flow is the rate at which vehicles (or persons) pass a point during a specified time period less than one hour, expressed as an equivalent hourly rate.

Demand is the number of vehicles (or persons) that desire to travel past a point during a specified period (also usually one hour). Demand is frequently higher than actual volumes where congestion exists. Some trips divert to alternative routes, while other trips are simply not made.

Capacity is the maximum rate at which vehicles can traverse a point or short segment during

a specified time period. It is a characteristic of the roadway. Actual volume can never be observed at levels higher than the true capacity of the section. However, such results may appear, because capacity is most often estimated using standard analysis procedures of the Highway Capacity Manual [ HCM ] . These estimates may indeed be too low for some locations.

Volume, Demand, and Capacity

Consider the illustration of Figure 1. It shows a classic "bottleneck" location on a freeway, in this case consisting of a major merge area. For each approaching leg, and for the downstream freeway section, the actual volume (v), the demand (D) , and the capacity (c) of the segment are given. Capacity is the primary constraint on the facility. As shown in Figure 1, the capacity is 2,000 veh/h/ln, so that the capacity of the two-lane approach legs are 4,000 veh/h each, while the capacity of the downstream freeway, which has three lanes, is

6,000 veh/h.

Figure 1: Volume, Demand, and Capacity at a Bottleneck Location.

Assuming that the stated capacities are correct, no volume in excess of these capacities can ever be counted.

Simply put, you can’t carry six gallons of water in a five-gallon bucket. Therefore, it is informative to consider what would be observed for the situation as described.

On Approach l, the true demand is 3,800 veh/h and the capacity is 4,000 veh/h. On Approach 2, the true demand is 3,600 veh/h and the capacity is also 4,000 veh/h. There is no capacity deficiency on either approach.

Downstream of the merge, however, the capacity is 6,000 veh/h, but the sum of the  approaching demands is 3,800+  3,600 = 7,400 veh/h. This exceeds the capacity

of the segment.

Given this scenario, what can we expect to observe?

Any volume count downstream of the merge cannot exceed 6,000 veh/h for as long as the illustrated conditions exist. A count of 6,000 veh/h is expected.

Because of the capacity deficiency downstream of the merge, a queue of vehicles will begin to form and propagate upstream on both approaches.

If a count of entering vehicles on both approaches is taken upstream of the forming queues, the true demand would be counted on each approach, assuming that there has been no diversion of vehicles, to alternative routes.

If a count of approaching vehicles is taken within the forming queues, they would be unstable in both time and space, but their total should not exceed 6,000 veh/h, the capacity of the downstream freeway section.

Demand Exceeding Capacity.

Volume Pattern.

Bottleneck on Observed Volumes.

Observed Hourly Pattern.

Observed Daily Variation.

Monthly Volume Variation.

Key Values from Figure Above.

Variation within A week.

Field Techniques for Volume Studies

There are a wide variety of techniques and equipment that can be used to count vehicles as they pass a point.

Despite this, most studies are conducted using relatively simple techniques and basic technology. Volume can literally be counted using a satellite. While this is technically feasible, the expense involved generally does not warrant such sophistication.

Many counts are still conducted by hand using field observers with mechanical hand-counters. There are two primary reasons for using manual count techniques for volume studies. Often, a quick count is needed on a deadline. If the need is to obtain PM peak hour vehicle counts at an intersection today or tomorrow, the expense and time spent in gathering and placing automated equipment at the site is often not justified. The second reason for manual counts is that there are a number of parameters that cannot be easily discerned using automated procedures. These include:

- Vehicle occupancy

- Turning movements at intersections

- Vehicle classification

There are multivariate detectors that can classify vehicles by length. If data is desired that separates taxis from passenger cars, however, manual observation is needed.

Road tubes with mechanical or electronic counters are still frequently used. The last option is the use of permanent detectors with remote monitoring of the site.

Manual Count

When manual counts are conducted using hand-counters, observers must periodically enter the data on field sheets. This, of course, requires that they stop counting for some period of time while they manually transfer the data from the counter(s) to paper. The count is therefore not continuous when such methodologies are used.

In order to obtain continuous count information on a common basis, short breaks are introduced into the counting procedure. Such breaks must be systematic and uniform for all observers. The system revolves around the count period of the study (i.e., the unit of time for which volumes are to be observed are recorded).

Common count periods are 5 minutes, 15 minutes, and 60 minutes, although (as will be seen) other times can be-and occasionally must be-used.

Intersection Volume Count.

Traffic Volume Count.

Data from an Illustrative Volume Study

Limited Network Volume Studies

Control counts. Control counts are taken at selected representative locations to measure and

quantify demand variation patterns in time. In general, control counts must be maintained continuously throughout the study period.

Coverage counts. Coverage counts are taken at all locations for which data is needed. They are conducted as samples, with each location being counted for only a portion of the study period, in accordance with a pre established sampling plan.

A Sample Network Volume Study ; Control Counts.

Estimating Vehicle Miles Traveled (VMT) on a Network

One output of most limited-network volume studies is an estimate of the total vehicle-miles traveled (VMT) on the network during the period of interest. The estimate is done roughly by assuming that a vehicle counted on a link travels the entire length of the link. This is a reasonable assumption, as some vehicles traveling only a portion of a link will be counted while others will not, depending upon whether they cross the count location. Using the sample network of the previous section, the 8-hour volume results of Table 1 and assuming

all links are 0.25 miles long, Table 2 illustrates the estimation of VMT. In this case, the estimate is the average 8-hour VMT for the three days of the study. It cannot be expanded into an estimate of annual VMT without knowing more about daily and seasonal variation patterns throughout the year.

Table (2): Data and Computations for a Three-Day Study Option.

Table (2): Estimation of Vehicle-Miles Traveled on a Limited Network: An Example.

Daily Variation Factors

Weekly Variation Factors

Specialized Counting Studies

Origin and Destination Counts

There are many instances in which normal point counts of vehicles must be supplemented with knowledge of the origins and destinations of the vehicles counted. In major regional planning applications, origin and destination studies involve massive home-interview efforts to establish regional travel patterns. In traffic applications, the scope of origin and destination counts are more limited.

Common applications include:

1. Weaving-area studies

2. Freeway studies

3. Major activity center studies

O-D Counts Example.

Sample Expansion of  O-D Data

Cordon Counts

A cordon is an imaginary boundary around a study area of interest. It is generally established to define a CBD or other major activity.

The: cordoned area must be large enough to define the full area of interest, yet small enough so that accumulation estimates will be useful for parking and other traffic planning purposes.

The: cordon is established to cross all streets and highways at midblock locations, to avoid the complexity of establishing whether turning vehicles, are entering or leaving the cordoned area.

The: cordon should be established to minimize the number of crossing points wherever possible. Natural or man-made barriers (e.g., rivers, railroads, limited access highways, and similar features) can be used as part of the cordon.

Cordoned areas should have relatively uniform land use. Accumulation estimates are used to estimate street capacity and parking needs. Large cordons encompassing different land-use activities will not be focused enough for these purposes.

Accumulation Computations for an Illustrative Cordon Study

Screen-Line Counts

Screen-line counts and volume studies are generally conducted as part of a larger regional origin-destination study involving home interviews as the principal methodology. In such regional planning studies, home interview responses constitute a small but detailed sample that is used to estimate the number of trips per day (or some other specified time interval) between defined transportation zones that have been established within the study region. Because home interview samples are small and because additional data is used to estimate trip patterns for those passing through the study area or having only a single trip-end within the study area, it is necessary to use some form of field observations to check on the accuracy of predicted movements.

Screen lines are convenient barriers cutting through the study area with only a limited number of crossing points. Rivers, railroads, limited-access highways, and other features make good screen lines.