Traffic Engineering

Spot Speed Studies

Spot Speed is the average speed of vehicles passing a point, or the time mean speed. Spot Speed studies are conducted to estimate the distribution of speeds of vehicles in a stream of traffic at a particular location on a highway. Carried out by recording the speeds of a sample of vehicles at a specified location.

Spot speeds have been previously defined as the average speed of vehicles passing a point on a highway. Recalling the basic discussion of speed measures , this is the time mean speed. As the traffic engineer is interested in conducting spot speed studies under conditions of free flow (i.e., observed speeds are not impeded by volume and density conditions), they are generally not conducted when volumes are in excess of 750-1,000 veh/h/ln on freeways or 500 veh/h/ln on other types of uninterrupted flow facilities.

Speed Definitions

Average or time mean speed-the average speed of all vehicles passing the study location during the period of the study.

Standard deviation:-in simplistic terms, the standard deviation of speeds is the average difference between observed speeds and the time mean speed during the period of the study.

85th percentile speed:-the speed below which 85% of the vehicles travel.

Median:-the speed that equally divides the distribution of spot speeds; 50% of observed speeds are higher than the median; 50% of observed speeds are lower than the median.

Pace:-a 10-mi/h increment in speeds that encompasses the highest proportion of observed speeds (as compared with any other 10-mi/h increment).

Uses of Spot Speed Data

1. Establishing the effectiveness of new or existing speed limits or enforcement practices.

2. Determining appropriate speed limits for application.

3. Establishing speed trends at the local, state, and national level to assess the effectiveness of national policy on speed limits and enforcement.

4. Specific design applications determining appropriate sight distances, relationships between speed and highway alignment, and speed performance with respect to steepness and length of grades.

5. Specific control applications for the timing of “yellow” and “all red” intervals for traffic signals, proper placement of signs, and development of appropriate signal progressions.

6. Investigation of high-accident locations at which speed is suspected to be a contributing cause to the accident experience.

Measurement Techniques

Spot speeds are generally measured using one of two techniques:

1. Measurement of travel times as vehicles traverse a short measured distance along the highway

2. Use of hand-held or fixed-mounted radar meters

Analysis of Spot Speed Data

The data summarized in the illustrative field sheet of Figure 1 represents, in raw form, the results of the spot speed study. To fully understand and describe the results, a number of techniques are used to both present and analyze the data in a systematic format.

Figure 1: Field Data for an Illustrative Spot Speed Study.

Frequency Distribution Table

The data of Figure 1 is reformatted into a frequency distribution table, shown in Table 1. This tabular array shows the total number of vehicles observed in each speed group. For the convenience of subsequent use, the table includes one speed group at each extreme for which no vehicles were observed. The “middle speed” (S) of the third column is taken as the midpoint value within the speed group. The use of this value will be discussed in a later section.

Table 1: Frequency Distribution Table for Illustrative Spot Speed Study

The fourth column of the table shows the number of vehicles observed in each speed group. This value is known as the frequency for the speed group. These values are taken directly from the field sheet of Figure 1. In the fifth column, the percentage of total observations in each speed group is computed as:

where: ni =: number of observations (frequency) in speed group i

N = total number of observations in the sample

For the 40-42 mi/h speed group, there are 13 observations in a total sample of 283 speeds. Thus, the percent frequency, %, is 100 * (13/283) = 4.6% for this group. The cumulative percent frequency (cum %) is the percentage of vehicles traveling at or below the highest speed in the speed group:

where: x = consecutive number (starting with the lowest speed group) of the speed group for which the cumulative percent frequency is desired.  For the 40-42 mi/h speed group, the sum of the frequencies for all speed groups having a high-speed boundary of 42 mi/h or less is found as 13 + 7 + 5 + 5 + O= 30. The cumulative percent frequency is then 100* (30/283) = 10.6%.

Frequency and Cumulative Frequency Distribution Curves

Figure 2. Frequency and Cumulative Frequency Distribution Curves for the Illustrative Spot Speed Study.

Figure 2. illustrates two standard plots that are generally prepared from the data summarized in the frequency

distribution table. These are:

Frequency distribution curve. For each speed group, the % frequency of observations within the group is plotted versus the middle speed of the group (S).

Cumulative frequency distribution curve. For each speed group, the % cumulative frequency of observations is plotted versus the high limit of the speed group.

Note that the two frequencies are plotted versus different speeds. The middle speed is used for the frequency distribution curve. The cumulative frequency distribution curve, however, results in a very useful plot of speed versus the percent of vehicles traveling at or below the designated speed. For this reason, the upper limit of the speed group is used as the plotting point.

Descriptive Statistics

Common descriptive statistics may be computed from the data in the frequency distribution table or determined graphically from the frequency and cumulative frequency distribution curves. These statistics are used to describe two important characteristics of the distribution:

Central tendency-measures that describe the approximate middle or center of the distribution.

Dispersion-measures that describe the extent to which data spreads around the center of the distribution.

Measures of central tendency include the average or mean speed, the median speed, the modal speed, and the pace. Measures of dispersion include the 85th and 15th percentile speeds and the standard deviation.

The Mean Speed: A Measure of Central Tendency

Easily found as the sum of the observed values divided by the number of observations. In a spot speed study, however, individual values of speed are not recorded; rather, the frequency of observations within defined speed groups is known. Computing the mean speed requires the assumption that the average speed within a given speed group is the middle speed, S, of the group.

This is the reason that speed groups of more than 5 mi/h are never used. This assumption becomes less valid as the size of the speed groups increases. For 2 mi/h speed groups, as in our illustrative study, the assumption is usually quite good. If this assumption is made, the sum of all speeds in a given speed group may be computed as:

niSi

where:

ni = frequency of observations in speed group i.

Si = middle speed of speed group i.

The sum of all speeds in the distribution may then be found by adding this product for all speed groups:

iniSi

The mean or average speed is then computed as the sum divided by the number of observed speeds:

where: X = average speed for the sample observations, mi/h

N = total sample size.

For the illustrative study data presented in Figure 1 and Table 1, the average or mean speed is:

The Median Speed: Another Measure of Central Tendency

The median speed is defined as the speed that divides the distribution into equal parts (Le., there are as many observations of speeds higher than the median as there are lower than the median). It is a positional value and is not affected by the absolute value of extreme observations.

The difference between the median and mean is best illustrated by example. Three speeds are observed:

30 mi/h, 40 mi/h, and 50 mi/h. Their average is (30 + 40 + 50)/3 = 40 mi/h. Their median is also 40 &, as it equally divides the distribution, with one speed higher than 40 mik and one speed lower than 40 mih. Another three speeds are then observed: 30 miih, 40 mih, and 70 mi/h.

Their average is (30 + 40 + 70)/3 = 46.7 mi/h. The median, however, is still 40 mi/h, with one speed higher and one speed lower than this observation. The mean is affected by the magnitude of the extreme observations; the median is affected only by the number of such observations.

As individual speeds have not been recorded in the illustrative study, however, the “middle value” is not easily determined from the tabular data of Table 9.1. It is easier to estimate the median graphically using the cumulative frequency distribution curve of Figure 9.4. By definition, the median equally divides the distribution.

Therefore, 50% of all observed speeds should be less than the median. This is exactly what the cumulative frequency distribution curve plots. If the curve is entered at 50% on the vertical axis, the median speed is found, as illustrated in Figure 2. For the illustrative study:

where P50 is the median or 50th percentile speed.

The Pace: Another Measure of Central Tendency

The pace is a traffic engineering measure not commonly used for other statistical analyses. It is defined as the 10-mi/h increment in speed in which the highest percentage of drivers is observed. It is also found graphically using the frequency distribution curve of Figure 2. The solution recognizes that the area under the frequency distribution curve between any two speeds approximates the percentage of vehicles traveling between those two speeds, where the total area under the curve is 100%.

The pace is found as follows: A lo-& template is scaled from the horizontal axis. Keeping this template horizontal, place an end on the lower left side of the curve and move slowly along the curve. When the right side of the template intersects the right side of the curve, the pace has been located. This procedure identifies the l0-mi/h increment that intersects the peak of the curve; this contains the most area and, therefore, the highest percentage of vehicles. The pace is shown in Figure 2 as:

43.5 - 53.5 mi/h

The Modal Speed: Another Measure of Central Tendency

The mode is defined as the single value of speed that is most likely to occur. As no discrete values were recorded, the modal speed is also determined graphically from the frequency distribution curve. A vertical line is dropped from the peak of the curve, with the result found on the horizontal axis. For the illustrative study, the modal speed is:

49.2 mi/h

The Standard Deviation: A Measure of Dispersion

The most common statistical measure of dispersion in a distribution is the standard deviation. It is a measure of how far data spreads around the mean value. In simple terms, the standard deviation is the average value of the difference between individual observations and the average value of those observations. Where discrete values of a variable are available, the equation for computing the standard1 deviation is:

where:

s =: the standard deviation

xi =: observation i

x =: average of all observations

N = number of observations

The difference between a given data point and the average is a direct measure of the magnitude of dispersion. These differences are squared to avoid positive and negative differences cancelling, and summed for all data points. They are then divided by N - 1. One statistical degree of freedom is lost because the mean of the distribution is known and used to compute the differences. If there are three numbers and it is known that the differences

between the values and the mean for the first two are “3” and “2,” then the third or last difference must be “-5,” as the sum of all differences must be zero. Only the first “N - 1” observations of differences are statistically random. A square root is taken of the results, as the values of the differences were squared to begin the computation.

Because discrete values of speed are not recorded, Equation 9-6 is modified to reflect group frequencies:

which may be manipulated into a more convenient form, as follows:

where all terms are as previously defined. This form is most convenient, as the first term is the sum of the last column of the frequency distribution table of Table 1. For the illustrative study, the standard deviation is:

Most observed speed distributions have standard deviations that are close to 5 mi&, as this represents most driver behaviour patterns reasonably well. Unlike averages and other central speeds, which vary widely from location to location, most speed studies yield similar standard deviations.

The 85th and 15th Percentile Speeds

The 85th and 15th percentile speeds give a general description of the high and low speeds observed by most reasonable drivers. It is generally thought that the upper and lower 15% of the distribution represents speeds that are either too fast or too slow for existing conditions. These values are found graphically from the cumulative frequency distribution curve of Figure 2. The curve is entered on the vertical axis at values of 85% and 15%. The respective

speeds are found on the horizontal axis, as shown in Figure 2. For the illustrative study, these speeds are:

The 85th and 15th percentile speeds can be used to roughly estimate the standard deviation of the distribution, although this is not recommended when the data is available for a precise determination:

In this case, the estimated value is close to the actual computed value of 4.96 mi/h.

The 85th and 15th percentile speeds give insight to both the central tendency and dispersion of the distribution.

As these values get closer to the mean, less dispersion exists and the stronger the central tendency of the distribution becomes.

Percent Vehicles Within the Pace

The pace itself is a measure of the center of the distribution. The percentage of vehicles traveling within the pace speeds is a measure of both central tendency and dispersion. The smaller the percentage of vehicles traveling within the pace, the greater the degree of dispersion in the distribution.

The percent of vehicles within the pace is found graphically using both the frequency distribution and cumulative frequency distribution curves of Figure 2. The pace speeds were determined previously from the frequency distribution curves. Lines from these speeds are dropped vertically to the cumulative frequency distribution curve. The percentage of vehicles traveling at or below each of these speeds can then be determined from the vertical axis of the cumulative frequency distribution curve, as shown. Then:

Even though speeds between 34 and 62 mi/h were observed in this study, over 70% of the vehicles traveled at speeds between 43.5 and 53.5 mi/h. This represents normal traffic behaviour with a standard deviation of approximately 5 mi/h.

Using the Normal Distribution in the Analysis of Spot Speed Data

If observed speeds are assumed to be normally distributed, then several additional analyses of the data may be conducted. Recall that the standard notation x:N[40,25] signifies that the variable “x” is normally distributed with a mean of “40” and a variance of “25.” The standard deviation is the square root of the variance, or “5” in this case. Recall also that a value of ‘‘Y on any normal distribution can be converted to an equivalent value of “z” on the standard normal distribution, where z:N[0,l]:

where:

xi = a value on any normal distribution

μ= the true mean of the distribution of values xi

σ= the true standard deviation of the distribution value of xi

zi= equivalent value on the standard normal distribution z:N[0,l]

In practical terms, the true values, μ and σ, are unknown. The results from a spot speed study are estimates of the true mean and standard deviation of the distribution based upon a measured sample, X and s.

Precision and Confidence Intervals

When a spot speed study is conducted, a single value of the mean speed is computed. For the illustrative study of this chapter, the mean is 48.1 mi/h, based upon a sample of 283 observations. In effect, this value, based upon a finite number of measured speeds, is being used to estimate the true mean of the underlying distribution of all vehicles traversing the site under uncongested conditions.

The number of such vehicles, for all practical and statistical purposes, is infinite. The measured value of x is being used as an estimate for μ. The first statistical question that must be answered is: How good is this estimate?

If a variable x is normally distributed:

it can be shown that the distribution of sample means (of a set of means with a constant sample size, n) is also normally distributed, as follows:

Assume that 100 speed observations had an average value of 50 mi/h. The speeds are then arranged in 10 groups of 10 speeds, and 10 separate averages are computed (one for each group). The average of the 10 group averages would still be 50 mi/h, as the mean of the distribution of sample means is the same as the mean of the original distribution. The standard deviations, however, would be different, as the grouping and averaging process significantly reduces the occurrence of extreme values. For example, in a distribution

with an average speed of 50 mi/h, it is conceivable that some observations of 70 mi/h or more would be obtained. However, at the same site, it is highly unlikely that the average of any 10 observed speeds  would be 70 mi/h or higher.

The standard error of the mean, E, is simply the standard deviation of a distribution of sample means

with a constant group size of n:

where:

E = standard error of the mean.

s = standard deviation of the original distribution of the individual values.

n = number of samples in each group of observation.

These characteristics, together with the standard error of the mean, can be used to quantify the quality of the sample estimate of the true mean of the underlying distribution. In effect, the entire illustrative spot speed study (with its sample size of 283 values) is considered to be a single point on a distribution of sample means, all with a group size of 283. Assuming a normal distribution, it is known that 95% of all values lie between the mean f 1.96 standard deviations; 99.7% of all values lie between the mean f3.00 standard deviations. Thus, it is 95% certain that the sample mean (48.1 m a ) is within the range of the true mean f 1.96 standard deviations. The standard deviation is, in this case, the standard error of the mean. Then:

95% of the time. The percentage is referred to as the confidence interval, while the precision of the measurement is given by the term 1.96 E. For the illustrative spot speed study:

Rounding off the values, it can be stated that we are 95% confident that the true mean of the underlying speed distribution lies between 47.5 and 48.7 mi/h. For a 99.7% confidence level:

Again rounding off these values, it can be stated that we are 99.7% confident that the true mean of the underlying speed distribution lies between 47.2 and 49.0 mi/h.

Estimating the Required Sample Size

While it is useful to know the confidence level and precision of a measured sample mean after the fact, it is more useful to determine what sample size is required to obtain a measurement that satisfies a predetermined precision and confidence level. Given that the precision or tolerance (e) of the estimate is the f range around the mean:

These equations can now be solved for the sample size, n. To obtain a desired precision with 95% confidence:

To obtain a desired precision with 99.7% confidence:

Table 2. Illustrative Example for Sample Size Computation.

Before and After Spot Speed Studies

There are many traffic engineering actions that can help reduce speeds, including lowered speed limits, stricter enforcement measures, warning signs. The major study issue, however, is to demonstrate that speeds have indeed been successfully reduced.

The statistical tests for the significance of observed differences have four possible results:

(1) the actual difference is significant, and the statistical test determines that it is significant;

(2) the actual difference is not significant, and the statistical test determines that it is not significant;

(3) the actual difference is significant and the statistical test determines that it is not significant;

(4) the actual difference is not significant and the statistical test determines that it is significant.

The normal approximation is applied by converting the observed reduction in mean speeds to a value of “z” on the standard normal distribution:

If the target speed lies within this range, it can be stated that it was successfully achieved.

Testing for Normalcy: The Chi-square Goodness-of- Fit Test

Virtually all of the statistical analyses of this section start with the basic assumption that the speed distribution can be mathematically represented as normal. For completeness, it is therefore necessary to conduct a statistical test to confirm that this assumption is correct.

Table 3. Chi-square Test for Normalcy on Illustrative Spot Speed Data.

Proper Location for Speed Studies

The location at which speed measurements are taken must conform to the intended purpose of the study. If approach speeds at a toll plaza are deemed to be too high, measurements should be taken at a point before drivers start to decelerate. Similarly, if excessive speed around a curve is thought to be contributing to off-the-road accidents, speed measurements should be taken in advance of the curve, before deceleration begins.

It may also be appropriate, however, to measure speeds at the point where accidents are occurring for comparison with approach speeds. This would allow the traffic engineer to assess whether the problem is excessive approach speed or that drivers are not decelerating sufficiently through the subject geometric element, or a combination of both. A study of intersection approach speeds must also be taken at a point before drivers begin to decelerate. This may be a moving point, given that queues get shorter and longer at different periods of

the day.