Traffic Engineering

Travel Time Studies

Travel-time studies involve significant lengths of a facility or group of facilities forming a route. Information on the travel time between key points within the study area is sought and is used to identify those segments in need of improvements. Travel-time studies are often coordinated with delay observations at points of congestion along the study route.

Travel-time information is used for many purposes, including:

1. To identify problem locations on facilities by virtue of high travel times and or delay.

2. To measure arterial level of service, based on average travel speeds and travel times.

3. To provide necessary input to traffic assignment models, which focus on link travel time as a key determinant of route selection.

4. To provide travel-time data for economic evaluation of transportation improvements.

5. To develop time contour maps and other depictions of traffic congestion in an area or region.

Field Study Techniques

1. Floating-car technique. In this technique, the test-car driver is asked to pass as many vehicles as pass the test car. In this way, the vehicle’s relative position in the traffic stream remains unchanged, and the test car approximates the behaviour of an average vehicle in the traffic stream.

2. Maximum-car technique. In this procedure, the driver is asked to drive as fast as is safely practical in the traffic stream without ever exceeding the design speed of the facility.

3. Average-car technique. The driver is instructed to drive at the approximate average speed of the traffic stream.

Travel Time Data Along an Arterial:

An Example of the Statistics of Travel Times

Given the cost and logistics of travel-time studies (test cars, drivers, multiple runs, multiple days of study, etc), there is a natural tendency to keep the number of observations, N, as small as possible. This case considers a hypothetical arterial on which the true mean running time is 196 seconds over a three-mile section. The standard deviation of the travel time is 15 seconds. The distribution of running times is normal. Note that the discussion is, at this point, limited to running times. These do not include stopped delays encountered along the route and are not equivalent to travel times, as will be seen.

Given the normal distribution of travel times, the mean travel time for the section is 196 seconds, and 95% of all travel times would fall within 1.96(15) = 29.4 seconds of this value. Thus, the 95% interval for travel times would be between 196 - 29.4 = 166.6 seconds and 196 + 29.4 = 225.4 seconds. The speeds corresponding to these travel times (including the average) are:

Table 1: A Sample Travel Time Field Sheet.

Note that the average of the two 95% confidence interval limits is (47.9 + 64.8)/2 = 56.4 mi/h, NOT 55.1 mi/h . This discrepancy is due to the fact that the travel times are normally distributed and are therefore symmetric. The resulting running speed distribution is skewed. The distribution of speeds, which are inverse to travel times, cannot be normal if the travel times are normal. The 55.1 mi/h value is the appropriate average speed, based on the observed average travel time over the three-mile study section.

So far, this discussion considers only the running times of test vehicles through the section. The actual travel time results of 20 test-car runs are illustrated in Figure 1.

Figure 1. Histogram of Hypothetical Travel Time Data for 20 Runs Over a Three-Mile Section.

This distribution does not look normal. In fact, it is not normal at all, as the total travel time represents the sum of running times (which are normally distributed) and stop time delay that follows another distribution entirely.

Specifically, it is postulated that:

The actual mean travel time of the observations in Figure 1 is 218.5 seconds, with a standard deviation of 38.3 seconds. The 95% confidence limits on the average are:

The speeds associated with these average and limiting travel times are:

Another way of addressing the average travel time is to add the average running time (196 s) to the average delay time,, which is computed from the probabilities noted above as:

The average travel time is then expected to be 196.0 +22.5 = 218.5 s, which is the same average obtained from the histogram of measurements.

Travel-Time Displays

Figure 2. A Travel Time Contour Map (Used with permission of Prentice Hall, Inc., from Pline, J., Editor, Traffic Engineering Handbook, 4th Edition, Institute of Transportation Engineers, Washington DC, 1992, pg. 69.)

Figure 2. A Plot of Elapsed Time versus Distance (Used with permission of Prentice Hall, Inc., from Pline, J., Editor, TrafJic Engineering Handbook, 4th Edition, Institute of  Transportation Engineers, Washington DC, 1992.)

Figure 3. Field Sheet for Signalized Intersection Delay Studies (Used with permission of Transportation Research Board, Highway Capacity Manual, 4th Edition, Washington DC, pg. 16-173.)