Parking Studies

Any vehicle traveling on a highway will at one time or another be parked for either a relatively short time or a much longer time, depending on the reason for parking. The provision of parking facilities is therefore an essential element of the highway mode of transportation. The need for parking spaces is usually very great in areas where land uses include business, residential, or commercial activities. The growing use of the automobile as a personal feeder service to transit systems (“park-and-ride”) has also increased the demand for parking spaces at transit stations. In areas of high density, where space is very expensive, the space provided for automobiles usually has to be divided between that allocated for their movement and that allocated for parking them.

Providing adequate parking space to meet the demand for parking in the CBD may necessitate the provision of parking bays along curbs which reduces the capacity of the streets and may affect the level of service. This problem usually confronts a city traffic engineer. The solution is not simple, since the allocation of available space will depend on the goals of the community which the traffic engineer must take into consideration when trying to solve the problem. Parking studies are therefore used to determine the demand for and the supply of parking facilities in an area, the projection of the demand, and the views of various interest groups on how best to solve the problem. Before we discuss the details of parking studies, it is necessary to discuss the different types of parking facilities.

Types of Parking Facilities

Parking facilities can be divided into two main groups: on-street and off-street.

1. On-Street Parking Facilities

These are also known as curb facilities. Parking bays are provided alongside the curb on one or both sides of the street. These bays can be unrestricted parking facilities if the duration of parking is unlimited and parking is free, or they can be restricted parking facilities if parking is limited to specific times of the day for a maximum duration.

Parking at restricted facilities may or may not be free. Restricted facilities also may be provided for specific purposes, such as to provide handicapped parking or as bus stops or loading bays.

On Street Parking

2. Off-Street Parking Facilities

These facilities may be privately or publicly owned; they include surface lots and garages. Self-parking garages require that drivers park their own automobiles; attendant-parking garages maintain personnel to park the automobiles.

Surface Car Parking

Multi-storey Car Park.

Roof  Parks.

Mechanical Parks.

Underground Parks.

Definitions of Parking Terms

1. A space-hour is a unit of parking that defines the use of a single parking space for a period of 1 hour.

2. Parking volume is the total number of vehicles that park in a study area during a specific length of time, usually a day.

3. Parking accumulation is the number of parked vehicles in a study area at any specified time. These data can be plotted as a curve of parking accumulation against time, which shows the variation of the parking accumulation during the day.

4. The parking load is the area under the accumulation curve between two specific times. It is usually given as the number of space-hours used during the specified period of time.

5. Parking duration is the length of time a vehicle is parked at a parking bay. When the parking duration is given as an average, it gives an indication of how frequently a parking space becomes available.

6. Parking turnover is the rate of use of a parking space. It is obtained by dividing the parking volume for a specified period by the number of parking spaces.

Methodology of Parking Studies

1. Inventory of Existing Parking Facilities

An inventory of existing parking facilities is a detailed listing of the location and all other relevant characteristics of each legal parking facility, private and public, in the study area. The inventory includes both on- and off-street facilities. The relevant characteristics usually listed include the following:

• Type and number of parking spaces at each parking facility

• Times of operation and limit on duration of parking, if any

• Type of ownership (private or public)

• Parking fees, if any, and method of collection

• Restrictions on use (open or closed to the public)

• Other restrictions, if any (such as loading and unloading zones, bus stops, or taxi ranks)

• Probable degree of permanency (can the facility be regarded as permanent or is it just a temporary facility?)

Collection of Parking Data

Accumulation: Accumulation data are obtained by checking the amount of parking during regular intervals on different days of the week. The checks are usually carried out on an hourly or 2-hour basis between 6:00 a.m. and 12 midnight. The selection of the times depends on the operation times of land-use activities that act as parking  generators.

For example, if a commercial zone is included, checks should be made during the times when retail shops are open, which may include periods up to 9:30 p.m. on some days. On the other hand, at truck stops, the highest accumulation may occur around midnight which requires information to be collected at that time. The information obtained is used to determine hourly variations of parking and peak periods of parking demand. (See Figure 1.) below:

Figure 1. Parking Accumulation at a Parking Lot.

Turnover and Duration: Information on turnover and duration is usually obtained by collecting data on a sample of parking spaces in a given block. This is done by recording the license plate of the vehicle parked on each parking space in the sample at the ends of fixed intervals during the study period. The length of the fixed intervals depends on the maximum permissible duration. For example, if the maximum permissible duration of parking at a curb face is 1 hour, a suitable interval is every 20 minutes. If the permissible duration is 2 hours, checking every 30 minutes would be appropriate. Turnover is then obtained from the equation:

Although the manual collection of parking data is still commonly used, it is now possible for all parking data to be collected electronically. Some of these electronic systems use wireless sensors to detect the arrival and departure of a vehicle at a parking space and the information sent to a central location through the internet. An example of this is the Spark Parking Inc. system. In addition to collecting data on parking, the Spark Parking System can be used to collect parking fees. The system provides for drivers to make calls soon after occupying a parking space from their mobile phones to record their credit cards and other personal information. The credit cards are then used for automatic payment of the parking fees. Figure 2 illustrates the general principles of the system.

Figure 2 The Spark Service System.

Parking Demand

Information on parking demand is obtained by interviewing drivers at the various parking facilities listed during the inventory. An effort should be made to interview all drivers using the parking facilities on a typical weekday between 8:00 a.m. and 10:00 p.m. Information sought should include (1) trip origin, (2) purpose of trip, and (3) driver’s destination after parking. The interviewer must also note the location of the parking facility, times of arrival and departure, and the vehicle type.

Parking interviews also can be carried out using the postcard technique, in which stamped postcards bearing the appropriate questions and a return address are handed to drivers or placed under windshield wipers. When this technique is used, usually only about 30 to 50 percent of the cards distributed are returned. It is therefore necessary to record the time and the number of cards distributed at each location, because this information is required to develop expansion factors, which are later used to expand the sample.

Analysis of Parking Data

Analysis of parking data includes summarizing, coding, and interpreting the data so that the relevant information required for decision making can be obtained. The relevant information includes the following:

• Number and duration for vehicles legally parked

• Number and duration for vehicles illegally parked

• Space-hours of demand for parking

• Supply of parking facilities

The analysis required to obtain information on the first two items is straightforward; it usually involves simple arithmetical and statistical calculations. Data obtained from these items are then used to determine parking space-hours.

The space-hours of demand for parking are obtained from the expression:

where

D = space vehicle-hours demand for a specific period of time.

N= number of classes of parking duration ranges.

ti = midparking duration of the ith class.

ni =number of vehicles parked for the ith duration range.

The space-hours of supply are obtained from the expression:

where

S = practical number of space-hours of supply for a specific period of time N = number of parking spaces available.

ti =total length of time in hours when the ith space can be legally parked on during the specific period.

f =efficiency factor.

The efficiency factor f is used to correct for time lost in each turnover. It is determined on the basis of the best performance a parking facility is expected to produce. Efficiency factors therefore should be determined for different types of parking facilities—for example, surface lots, curb parking, and garages. Efficiency factors for curb parking, during highest demand, vary from 78 percent to 96 percent; for surface lots and garages, from 75 percent to 92 percent. Average values of f are 90 percent for curb parking, 80 percent for garages, and 85 percent for surface lots.

Example

The owner of a parking garage located in a CBD has observed that 20% of those wishing to park are turned back every day during the open hours of 8 a.m. to 6 p.m. because of lack of parking spaces. An analysis of data collected at the garage indicates that 60% of those who park are commuters, with an average parking duration of 9 hr, and the remaining are shoppers, whose average parking duration is 2 hr. If 20% of those who cannot park are commuters and the rest are shoppers, and a total of 200 vehicles currently park daily in the garage, determine the number of additional spaces required to meet the excess demand. Assume parking efficiency is 0.90.

Solution

Commuters now being served = 0.6 * 200 * 9 = 1080 space-hr

Shoppers now being served = 0.4 * 200 * 2 = 160 space-hr

Total number of vehicles turned away = 200 /0.8_ 200 = 50

Commuters not being served = 0.2 * 50 * 9 = 90 space-hr

Shoppers not being served = 0.8 * 50 * 2 = 80 space-hr

Total space-hours of demand = (1080 + 160 + 90 + 80) = 1410

Total space-hours served = 1080+ 160 = 1240

Number of space-hours required = 1410 _ 1240 = 170

S = 170

Use the length of time each space can be legally parked on (8 a.m. through 6 p.m. = 10 hr) to determine the number of additional spaces.

0.9 *10 * N = 170

N = 18.89

At least 19 additional spaces will be required, since a fraction of a space cannot be used.

Parking Stall Layout Considerations

The objective of the layout design is to maximize the number of stalls, while following the guidelines below.

- The layout of the parking facility must be flexible enough to adapt to future changes in vehicle dimensions.

- The stall and aisle dimensions must be compatible with the type of operation planned for the facility.

The critical dimensions are the width and length of stalls, the width of aisles, the angle of parking, and the radius of turns. All of these dimensions are related to the vehicle dimensions and performance characteristics. In recent years there have been a number of changes in vehicle dimensions. The popularity of minivans and sport utility vehicles has had an impact on the design of parking facilities. For the near future, a wide mix of vehicle sizes should be anticipated. There are three approaches for handling the layout:

- Design all spaces for large-size vehicles (about 6 feet wide and 17-18 ft long).

- Design some of the spaces for large vehicles and some for small vehicles (these are about 5 ft wide and 14-15 ft long).

- Provide a layout with intermediate dimensions (too small for large vehicles and too big for small vehicles).

For design, it is customary to work with stalls and aisles in combinations called "modules". A complete module is one access aisle servicing a row of parking on each side of the aisle. The width of an aisle is usually 12 to 26 feet depending on the angle at which the parking stalls are oriented.

Stall Width

For simplicity, the stall width is measured perpendicular to the vehicle, not parallel to the aisle. If the stall is placed at an angle of less than 90o, then the width parallel to the aisle will increase while the width perpendicular to the vehicle will remain the same.

Stall Length

The length of the stall should be large enough to accommodate most of the vehicles. The length of the stall refers to the longitudinal dimension of the stall. When the stall is rotated an angle of less than 90o, the stall depth perpendicular to the aisle increases up to 1 foot or more. It should be noted that the effective stall depth depends on the boundary conditions of the module, which could include walls on each side of the module, curbs with or without overhang, or drive-in versus back-in operations. For parking at angles of less than 90o, front bumper overhangs beyond the curbing are generally reduced with decreasing angle and, for example, drop to about 2 feet at 45o angles.

Angular Parking

Perpendicular Parking

Parallel Parking

Dimensions for Standard Parking Spaces

Figure 2: Dimensional Elements of Parking Modules.

Table 1: Stall Width Design Criteria for Various Parking Classifications.

Table 2: Parking Module Layout Dimension Guidelines.