Traffic Engineering

What is A Traffic: Traffic can be defined as the movement of pedestrians and goods along a route, and in the 21st century the biggest problem and challenge for the traffic engineer is often the imbalance between the amount of traffic and the capacity of the route, leading to congestion.

What is Traffic Engineering?

Traffic engineering is that phase of engineering that deals with the planning, geometric design and traffic operations of roads, streets and highways and their networks, terminals, abutting lands and relationships with other modes of transportation for the achievement of safe, efficient and convenient movement of people and goods.

Traffic engineering applies engineering principles that help solve transportation problems by considering the psychology and habits of the transportation system users.

2. Traffic Engineering Studies

Traffic studies may be grouped into three main categories:

(1) inventories,

(2) administrative studies, and

(3) dynamic studies.

Inventories provide a list or graphic display of existing information, such as street widths, parking spaces, transit routes, traffic regulations, and so forth.

Some inventories—for example, available parking spaces and traffic regulations—change frequently and therefore require periodic updating; others, such as street widths, do not.

Administrative studies use existing engineering records, available in government agencies and departments. This information is used to prepare an inventory of the relevant data. Inventories may be recorded in files but are usually recorded in automated data processing (ADP) systems. Administrative studies include the results of surveys, which may involve field measurements and/or aerial photography.

Dynamic traffic studies involve the collection of data under operational conditions and include:

 Traffic volume study.

 Speed study.

 Spot speed study.

 Speed and delay study.

 Origin and destination study.

 Traffic flow characteristics.

 Traffic capacity study.

 Parking study.

 Accident studies.

Basic Term Definition

Volume is the number of vehicles (or persons) passing a point during a specified time period,

which is usually one hour, but need not be.

Rate of flow is the rate at which vehicles (or persons) pass a point during a specified time period less than one hour, expressed as an equivalent hourly rate.

Demand is the number of vehicles (or persons) that desire to travel past a point during a specified period (also usually one hour). Demand is frequently higher than actual volumes where congestion exists. Some trips divert to alternative routes, while other trips are simply not made.

Capacity is the maximum rate at which vehicles can traverse a point or short segment during a specified time period. It is a characteristic of the roadway. Actual volume can never be observed at levels higher than the true capacity of the section. However, such results may appear, because capacity is most often estimated using standard analysis procedures of the Highway Capacity Manual (HCM).

Peak Hour Volume (PHV) is the maximum number of vehicles that pass a point on a highway during a period of 60 consecutive minutes. PHVs are used for:

a. Functional classification of highways.

b. Design of the geometric characteristics of a highway, for example, number of

lanes, intersection signalization, or channelization.

c. Capacity analysis.

d. Development of programs related to traffic operations, for example, one-way.

street systems or traffic routing.

e. Development of parking regulations.

Vehicle Classification (VC) records volume with respect to the type of vehicles,

for example, passenger cars, two-axle trucks, or three-axle trucks. VC is used in:

a. Design of geometric characteristics, with particular reference to turning-radii

requirements, maximum grades, lane widths, and so forth.

b. Capacity analyses, with respect to passenger-car equivalents of trucks.

c. Adjustment of traffic counts obtained by machines.

d. Structural design of highway pavements, bridges, and so forth.

Vehicle Miles of Travel (VMT) is a measure of travel along a section of road. It is the product of the traffic volume (that is, average weekday volume or ADT) and the length of roadway in miles to which the volume is applicable. VMTs are used mainly as a base for allocating resources for maintenance and improvement of highways.

Traffic Volume Studies

Traffic volume study is producer to determine mainly the volume of traffic moving on roads on a particular section during a particular time. The commonly units are vehicles/day or vehicles/hour

- AADT: Average annual daily traffic obtain by adding daily traffic count in one year divided by 365.

- ADT: Average daily traffic is the volume of traffic total counted on the roadway(two way) over a given time period(greater than one day but less than one year) divided by the number of days in that time  period.

Object and Uses of Traffic Volume Study:

- It is generally accepted as a true measure of the relative importance of roads and in deciding the priority for improvement and expansion.

- It is used in planning, traffic operation and control of existing facilities and also for planning the new facilities.

- It is used in the analysis of traffic patterns and trends.

- Useful in structural design of pavement.

- Pedestrian traffic volume study is used for planning side walk, cross walks, subway and pedestrian signals.

scope of traffic volume study

junction

Methodology

Automatic counting method

In this method, vehicles are counted automatically without any human involvement.

There are two techniques of automatic counting:

1. Contact system based:-

It is based on pneumatic, mechanical, magnetic or piezo-electric method.

2. Contactless system based:-

It is based on electrical/optical, ultrasound/infrared radar, micro wave, CCTV/video image processing method.

Equipments used for automatic counting methods

Vehicles numbers are recorder using ultrasound, light beam or other infrared technology.

Manual counting method

There are two types of manual counting

- Direct method: Data is counted by using hand tally manual counter.

- Indirect method: In this method, data is collected using video camera. Video is captured

for long time and data is collected later by rewinding.

- Video cameras are mounted to record the traffic condition on road.

Data presentation

The collect from traffic volume counts may be presented in the several ways depending on the type of count conducted and primary use of the data.

- Trend chart showing volume trends over period of years are prepared. These data useful for planning future expansion, design and regulation.

- Variation chart showing hourly, daily and seasonal variations are also prepared. These help in deciding the facilities and regulation needed during peak traffic periods.

- Traffic flow maps along the routes are drawn. These helps to find the traffic volume distribution at glance.

- Volume flow diagram at intersection either drawn to a certain scale or indicating traffic volume as shown in this figure and it showing the details of crossing and turning traffic, these data is needed for intersection design.

Intersection Summary Sheet.

Traffic Flow Maps

These maps show traffic volumes on individual routes. The volume of traffic on each route is represented by the width of a band, which is drawn in proportion to the traffic volume it represents, providing a graphic representation of the different volumes that facilitates easy visualization of the relative volumes of traffic on different routes.

When flows are significantly different in opposite directions on a particular street or highway, it is advisable to provide a separate band for each direction. In order to increase the usefulness of such maps, the numerical value represented by each band is listed near the band. Figure below shows a typical traffic flow map.

Fig. Example of a Traffic Flow Map.

Time-Based Distribution Charts

These charts show the hourly, daily, monthly, or annual variations in traffic volume in an area or on a particular highway. Each volume is usually given as a percentage of the average volume. Figure below shows typical charts for monthly, daily, and hourly variations.

Fig.  Traffic Volumes on an Urban Highway.

Summary Tables

These tables give a summary of traffic volume data such as PHV, Vehicle Classification (VC), and ADT in tabular form. Table below . is a typical summary table.

Table  Summary of Traffic Volume Data for a Highway Section.

Passenger Car Unit

The different vehicle classes have a wide range of statics characteristics and dynamic characteristics, apart from these the driver behavior of the different vehicle classes is also found to vary considerable. Therefore mixed traffic flow characteristics are very much complex when compare to homogeneous traffic and it is difficult to estimate the traffic volume, capacity of roadway under the mixed traffic flow, unless the different vehicle classes are converted to one

common standard vehicle unit.

•Therefore it is a common practice to consider the passenger car as the standard vehicle unit to convert the other vehicle classes and this unit is called passenger car unit PCU.

Passenger Car Unit :Passenger Car Unit (PCU) is a metric used in Transportation Engineering, to assess traffic-flow rate on a highway. A Passenger Car Unit is a measure of the impact that a mode of transport has on traffic variables (such as headway, speed, density) compared to a single standard passenger car. This is also known as passenger car equivalent. For example, typical values of PCU (or PCE) are: Highway capacity is measured in PCU/hour daily.

It is a common practice to consider the passenger car as the standard vehicle unit to convert the

other vehicle classes and this unit is called Passenger car unit or PCU.

Thus in mixed traffic flow, the traffic volume & capacity are generally expressed as PCU per hour or PCU/lane/hour & the traffic density as PCU per kilometer length of lane.

Table: Typical Values of PCU

| Car | 1.0 |
| --- | --- |
| Motorcycle | 0.5 |
| Bicycle | 0.2 |
| Bus, Truck | 3.5 |
| 3-wheeler | 0.8 |

Factor affecting Passenger Car Unit

- Vehicle characteristics

- Speed distribution of the mixed traffic stream, volume to capacity

- ratio.

- Roadway characteristics.

- Regulation and control of traffic.

- Environmental and climatic conditions.

Types of Volume Counts

Different types of traffic counts are carried out, depending on the anticipated use of the data to be collected.

1. Cordon Counts

When information is required on vehicle accumulation within an area such as the central business district (CBD) of a city, particularly during a specific time, a cordon count is undertaken. The area for which the data are required is cordoned off by an imaginary closed loop; the area enclosed within this loop is defined as the cordon area.

Figure below shows such an area where the CBD of a city is enclosed by the imaginary loop ABCDA. The intersection of each street crossing the cordon line is taken as a count station; volume counts of vehicles and/or persons entering and leaving the cordon area are taken. The information obtained from such a count is useful for planning parking facilities, updating and evaluating traffic operational techniques, and making long-range plans for freeway and arterial street systems.

Fig. Example of Station Locations for a Cordon Count.

2. Screen Line Counts

In screen line counts, the study area is divided into large sections by running imaginary lines, known as screen lines, across it. In some cases, natural and manmade barriers, such as rivers or railway tracks, are used as screen lines. Traffic counts are then taken at each point where a road crosses the screen line. It is usual for the screen lines to be designed or chosen such that they are not crossed more than once by the same street. Collection of data at these screen-line stations at regular intervals facilitates the detection of variations in the traffic volume and traffic flow direction due to changes in the land-use pattern of the area.

3. Intersection Counts

Intersection counts are taken to determine vehicle classifications, through movements, and turning movements at intersections. These data are used mainly in determining phase lengths and cycle times for signalized intersections, in the design of channelization at intersections, and in the general design of improvements to intersections.

4. Pedestrian Volume Counts

Volume counts of pedestrians are made at locations such as subway stations, midblocks, and crosswalks. The counts are usually taken at these locations when the evaluation of existing or proposed pedestrian facilities is to be undertaken. Such facilities may include pedestrian overpasses or underpasses. These counts can be used for crash analysis, capacity analysis, and determining minimum signal timings at signalized intersections.

5. Periodic Volume Counts

In order to obtain certain traffic volume data, such as AADT, it is necessary to obtain data continuously. However, it is not feasible to collect continuous data on all roads because of the cost involved. To make reasonable estimates of annual traffic volume characteristics on an area-wide basis, different types of periodic counts, with count durations ranging from 15 minutes to continuous, are conducted; the data from these different periodic counts are used to determine values that are then employed in the estimation of annual traffic characteristics. The periodic counts usually conducted are continuous, control, or coverage counts.

6. Continuous Counts

These counts are taken continuously using mechanical or electronic counters. Stations at which continuous counts are taken are known as permanent count stations. In selecting permanent count stations, the highways within the study area must first be properly classified. Each class should consist of highway links with similar traffic patterns and characteristics. A highway link is defined for traffic count purposes as a homogeneous section that has the same traffic characteristics, such as AADT and daily, weekly, and seasonal variations in traffic volumes at each point. Broad classification systems for major roads may include freeways, expressways, and major arterials. For minor roads, classifications may include residential, commercial, and industrial streets.

7. Control Counts

These counts are taken at stations known as control-count stations, which are strategically located so that representative samples of traffic volume can be taken on each type of highway or street in an area-wide traffic counting program. The data obtained from control counts are used to determine seasonal and monthly variations of traffic characteristics so that expansion factors can be determined. These expansion factors are used to determine year-round average values from short counts.

Control counts can be divided into major and minor control counts:

a) Major control counts : are taken monthly, with 24-hour directional counts taken on at least three days during the week (Tuesday, Wednesday, and Thursday) and also on Saturday and Sunday to obtain information on weekend volumes. It is usual to locate at least one major control-count station on every major street. The data collected give information regarding hourly, monthly, and seasonal variations of traffic characteristics.

b) Minor control counts: are five-day weekday counts taken every other month on minor roads.

8. Coverage Counts

These counts are used to estimate ADT, using expansion factors developed from control counts. The study area is usually divided into zones that have similar traffic characteristics. At least one coverage count station is located in each zone. A 24-hour non-directional weekday count is taken at least once every four years at each coverage station. The data indicate changes in area-wide traffic characteristics.

Table  Data and Computations for a One-Day Network Volume Study.

Adjustment of Periodic Counts

Expansion factors, used to adjust periodic counts, are determined either from continuous count stations or from control count stations.

1. Expansion Factors from Continuous Count Stations

Hourly, daily, and monthly expansion factors can be determined using data obtained at continuous count stations.

Hourly expansion factors (HEFs) are determined by the formula:

These factors are used to expand counts of durations shorter than 24 hour to 24-hour volumes by multiplying the hourly volume for each hour during the count period by the HEF for that hour and finding the mean of these products.

Daily expansion factors (DEFs) are computed as:

These factors are used to determine weekly volumes from counts of 24-hour duration by multiplying the 24-hour volume by the DEF.

Monthly expansion factors (MEFs) are computed as:

The AADT for a given year may be obtained from the ADT for a given month by multiplying this volume by the MEF.

Tables 1, 2, and 3 give expansion factors for a particular primary road in Virginia. Such expansion factors should be determined for each class of road in the classification system established for an area.

Table (1) Hourly Expansion Factors for a Rural Primary Road.

Table (2) Daily Expansion Factors for a Rural Primary Road.

Table (3) Monthly Expansion Factors for a Rural Primary Road.

Example (1): Calculating AADT Using Expansion Factors

Estimating Vehicle Miles Traveled (VMT) on a Network

One output of most limited-network volume studies is an estimate of the total vehicle-miles traveled (VMT) on the network during the period of interest. The estimate is done roughly by assuming that a vehicle counted on a link travels the entire length of the link.

Table (4) illustrates the estimation of VMT. In this case, the estimate is the average 8-hour VMT for the three days of the study. It cannot be expanded into an estimate of annual VMT without knowing more about daily and seasonal variation patterns throughout the year.

Table (4): Estimation of Vehicle-Miles Traveled on a Limited Network: An Example