Traffic Engineering

Spot Speed Study

Spot Speed Study

Spot speed studies are conducted to estimate the distribution of speeds of vehicles in a stream of traffic at a particular location on a highway. The speed of a vehicle is defined as the rate of movement of the vehicle; it is usually expressed in miles per hour (mi/h) or kilometres per hour (km/h). A spot speed study is carried out by recording the speeds of a sample of vehicles at a specified location. Speed characteristics identified by such a study will be valid only for the traffic and environmental conditions that exist at the time of the study.

Spot Speed: Instantaneous speed of a vehicle at a specified location.

Average Speed: Average of the spot speeds of all vehicles passing a given point on the highway.

Running Speed: Average speed maintained by a vehicle over a given course while the vehicle is in motion.

Running speed= Length of course / Running time

= Length of course /( Journey time- Delays)

Journey speed

Also known as overall travel speed  It is the effective speed between two points. It is the distance between two points divided by the total time taken by the vehicle to complete the journey ,including all delays.

Journey speed = Distance/ Total journey Time (including Delays).

Speed characteristics determined from a spot speed study may be used to:

1. Establish parameters for traffic operation and control, such as speed zones, speed limits (85th-percentile speed is commonly used as the speed limit on a road), and passing restrictions.

2. Evaluate the effectiveness of traffic control devices, such as variable message signs at work zones.

3. Monitor the effect of speed enforcement programs, such as the use of drone radar and the use of differential speed limits for passenger cars and trucks.

4. Evaluate and or determine the adequacy of highway geometric characteristics, such as radii of horizontal curves and lengths of vertical curves.

5. Evaluate the effect of speed on highway safety through the analysis of crash data for different speed characteristics.

6. Determine speed trends.

7. Determine whether complaints about speeding are valid.

As the traffic engineer is interested in conducting spot speed studies under conditions of free flow (i.e, observed speeds are not impeded by volume and density conditions), they are generally not conducted when volumes are in excess of 750-1,000 veh/h/ln on freeways or 500 veh/h/ln on other types of uninterrupted flow facilities.

Location of Spot Speed Studies

The following locations generally are used for the different applications listed:

1. Locations that represent different traffic conditions on a highway or highways are

used for basic data collection.

2. Mid-blocks of urban highways and straight, level sections of rural highways are

sites for speed trend analyses.

3. Any location may be used for the solution of a specific traffic engineering problem.

When spot speed studies are being conducted, it is important that unbiased data be obtained. This requires that drivers be unaware that such a study is being conducted.

Equipment used therefore, should be concealed from the driver, and observers conducting the study should be inconspicuous. Since the speeds recorded eventually will be subjected to statistical analysis, it is important that a statistically adequate number of vehicle speeds be recorded.

Methods of Measuring Speed

The methods available for measuring spot speed can be grouped as:

A. Those observations that require time taken by a vehicle to cover a known distance.

- It consist of:

- long base methods- vehicles are timed over a long distance.

- Short base methods- vehicles are timed over a Short distance, say about 2m.

B. Radar Speedometer which automatically records instantaneous speed.

C. Photographic method.

Those observations that require time taken by a vehicle to cover a known distance.

D. Long base methods

- Direct timing Procedure

- Enoscope.

Direct timing Procedure:

- Simplest method

- Two reference points are marked on the pavement at a suitable distance apart and an observer starts and stops stopwatch as a vehicle crosses these two marks.

- From known distance and time, speed can be calculated.

- Disadvantage: errors due to parallax effect.

- Observation by two observers one at each reference points

- First observer standing at the point in which the vehicle pass first, signals and seconds observer starts the stop watch. The second observer stops the stop watch when he observes the same vehicle passing the reference point.

- Disadvantage: reaction time of two observers.

Enoscope:

- Also known as mirror box.

- L-shaped box, open at both ends ,with a mirror set at 45 ˚ to the arms of the instrument.

- The instrument is placed directly opposite to the first reference point and the observer stations himself at the second reference point.

- Stop watch is started as soon as the vehicle passes the first point and is stopped as soon as it passes the observer.

- If two enoscope is used, the observer stations himself midway between two reference points.

Advantages:

- Eliminates parallax error.

- Equipment is of low cost, simple and can set up easily.

Analysis of Spot Speed Data

The data summarized in the illustrative field sheet of Figure 1 represents, in raw form, the results of the spot speed study. To fully understand and describe the results, a number of techniques are used to both present and analyze the data in a systematic format.

Figure 1: Field Data for an Illustrative Spot Speed Study.

Frequency Distribution Table

The data of Figure 1 is reformatted into a frequency distribution table, shown in Table 1. This tabular array shows the total number of vehicles observed in each speed group. For the convenience of subsequent use, the table includes one speed group at each extreme for which no vehicles were observed. The “middle speed” (S) of the third column is taken as the midpoint value within the speed group. The use of this value will be discussed in a later section.

Table 1: Frequency Distribution Table for Illustrative Spot Speed Study

The fourth column of the table shows the number of vehicles observed in each speed group. This value is known as the frequency for the speed group. These values are taken directly from the field sheet of Figure 1. In the fifth column, the percentage of total observations in each speed group is computed as:

where: ni =: number of observations (frequency) in speed group i

N = total number of observations in the sample

For the 40-42 mi/h speed group, there are 13 observations in a total sample of 283 speeds. Thus, the percent frequency, %, is 100 * (13/283) = 4.6% for this group. The cumulative percent frequency (cum %) is the percentage of vehicles traveling at or below the highest speed in the speed group:

where: x = consecutive number (starting with the lowest speed group) of the speed group for which the cumulative percent frequency is desired.  For the 40-42 mi/h speed group, the sum of the frequencies for all speed groups having a high-speed boundary of 42 mi/h or less is found as 13 + 7 + 5 + 5 + O= 30. The cumulative percent frequency is then 100* (30/283) = 10.6%.

Frequency and Cumulative Frequency Distribution Curves

Figure 2. Frequency and Cumulative Frequency Distribution Curves for the Illustrative Spot Speed Study.

Figure 2. illustrates two standard plots that are generally prepared from the data summarized in the frequency distribution table. These are:

Frequency distribution curve. For each speed group, the % frequency of observations within the group is plotted versus the middle speed of the group (S).

Cumulative frequency distribution curve. For each speed group, the % cumulative frequency of observations is plotted versus the high limit of the speed group.

Note that the two frequencies are plotted versus different speeds. The middle speed is used for the frequency distribution curve. The cumulative frequency distribution curve, however, results in a very useful plot of speed versus the percent of vehicles traveling at or below the designated speed. For this reason, the upper limit of the speed group is used as the plotting point.

Descriptive Statistics

Common descriptive statistics may be computed from the data in the frequency distribution table or determined graphically from the frequency and cumulative frequency distribution curves. These statistics are used to describe two important characteristics of the distribution:

Central tendency-measures that describe the approximate middle or center of the distribution.

Dispersion-measures that describe the extent to which data spreads around the center of the distribution.

Measures of central tendency include the average or mean speed, the median speed, the modal speed, and the pace. Measures of dispersion include the 85th and 15th percentile speeds and the standard deviation.

The Mean Speed: A Measure of Central Tendency

Easily found as the sum of the observed values divided by the number of observations. In a spot speed study, however, individual values of speed are not recorded; rather, the frequency of observations within defined speed groups is known. Computing the mean speed requires the assumption that the average speed within a given speed group is the middle speed, S, of the group.

This is the reason that speed groups of more than 5 mi/h are never used. This assumption becomes less valid as the size of the speed groups increases. For 2 mi/h speed groups, as in our illustrative study, the assumption is usually quite good. If this assumption is made, the sum of all speeds in a given speed group may be computed as:

niSi

where:

ni = frequency of observations in speed group i.

Si = middle speed of speed group i.

The sum of all speeds in the distribution may then be found by adding this product for all speed groups:

iniSi

The mean or average speed is then computed as the sum divided by the number of observed speeds:

where: X = average speed for the sample observations, mi/h

N = total sample size.

For the illustrative study data presented in Figure 1 and Table 1, the average or mean speed is:

The Median Speed: Another Measure of Central Tendency

The median speed is defined as the speed that divides the distribution into equal parts (Le., there are as many observations of speeds higher than the median as there are lower than the median). It is a positional value and is not affected by the absolute value of extreme observations.

The difference between the median and mean is best illustrated by example. Three speeds are observed:

30 mi/h, 40 mi/h, and 50 mi/h. Their average is (30 + 40 + 50)/3 = 40 mi/h. Their median is also 40 &, as it equally divides the distribution, with one speed higher than 40 mik and one speed lower than 40 mih. Another three speeds are then observed: 30 miih, 40 mih, and 70 mi/h.

Their average is (30 + 40 + 70)/3 = 46.7 mi/h. The median, however, is still 40 mi/h, with one speed higher and one speed lower than this observation. The mean is affected by the magnitude of the extreme observations; the median is affected only by the number of such observations.

As individual speeds have not been recorded in the illustrative study, however, the “middle value” is not easily determined from the tabular data of Table 9.1. It is easier to estimate the median graphically using the cumulative frequency distribution curve of Figure 9.4. By definition, the median equally divides the distribution.

Therefore, 50% of all observed speeds should be less than the median. This is exactly what the cumulative frequency distribution curve plots. If the curve is entered at 50% on the vertical axis, the median speed is found, as illustrated in Figure 2. For the illustrative study:

where P50 is the median or 50th percentile speed.

The Pace: Another Measure of Central Tendency

The pace is a traffic engineering measure not commonly used for other statistical analyses. It is defined as the 10-mi/h increment in speed in which the highest percentage of drivers is observed. It is also found graphically using the frequency distribution curve of Figure 2. The solution recognizes that the area under the frequency distribution curve between any two speeds approximates the percentage of vehicles traveling between those two speeds, where the total area under the curve is 100%.

The pace is found as follows: A lo-& template is scaled from the horizontal axis. Keeping this template horizontal, place an end on the lower left side of the curve and move slowly along the curve. When the right side of the template intersects the right side of the curve, the pace has been located. This procedure identifies the l0-mi/h increment that intersects the peak of the curve; this contains the most area and, therefore, the highest percentage of vehicles. The pace is shown in Figure 2 as:

43.5 - 53.5 mi/h

The Modal Speed: Another Measure of Central Tendency

The mode is defined as the single value of speed that is most likely to occur. As no discrete values were recorded, the modal speed is also determined graphically from the frequency distribution curve. A vertical line is dropped from the peak of the curve, with the result found on the horizontal axis. For the illustrative study, the modal speed is:

49.2 mi/h

The Standard Deviation: A Measure of Dispersion

The most common statistical measure of dispersion in a distribution is the standard deviation. It is a measure of how far data spreads around the mean value. In simple terms, the standard deviation is the average value of the difference between individual observations and the average value of those observations. Where discrete values of a variable are available, the equation for computing the standard1 deviation is:

where:

s =: the standard deviation

xi =: observation i

x =: average of all observations

N = number of observations

The difference between a given data point and the average is a direct measure of the magnitude of dispersion. These differences are squared to avoid positive and negative differences cancelling, and summed for all data points. They are then divided by N - 1. One statistical degree of freedom is lost because the mean of the distribution is known and used to compute the differences. If there are three numbers and it is known that the differences

between the values and the mean for the first two are “3” and “2,” then the third or last difference must be “-5,” as the sum of all differences must be zero. Only the first “N - 1” observations of differences are statistically random. A square root is taken of the results, as the values of the differences were squared to begin the computation.

Because discrete values of speed are not recorded, Equation 9-6 is modified to reflect group frequencies:

which may be manipulated into a more convenient form, as follows:

where all terms are as previously defined. This form is most convenient, as the first term is the sum of the last column of the frequency distribution table of Table 1. For the illustrative study, the standard deviation is:

Most observed speed distributions have standard deviations that are close to 5 mi&, as this represents most driver behaviour patterns reasonably well. Unlike averages and other central speeds, which vary widely from location to location, most speed studies yield similar standard deviations.

The 85th and 15th Percentile Speeds

The 85th and 15th percentile speeds give a general description of the high and low speeds observed by most reasonable drivers. It is generally thought that the upper and lower 15% of the distribution represents speeds that are either too fast or too slow for existing conditions. These values are found graphically from the cumulative frequency distribution curve of Figure 2. The curve is entered on the vertical axis at values of 85% and 15%. The respective

speeds are found on the horizontal axis, as shown in Figure 2. For the illustrative study, these speeds are:

The 85th and 15th percentile speeds can be used to roughly estimate the standard deviation of the distribution, although this is not recommended when the data is available for a precise determination:

In this case, the estimated value is close to the actual computed value of 4.96 mi/h.

The 85th and 15th percentile speeds give insight to both the central tendency and dispersion of the distribution.

As these values get closer to the mean, less dispersion exists and the stronger the central tendency of the distribution becomes.

Percent Vehicles Within the Pace

The pace itself is a measure of the center of the distribution. The percentage of vehicles traveling within the pace speeds is a measure of both central tendency and dispersion. The smaller the percentage of vehicles traveling within the pace, the greater the degree of dispersion in the distribution.

The percent of vehicles within the pace is found graphically using both the frequency distribution and cumulative frequency distribution curves of Figure 2. The pace speeds were determined previously from the frequency distribution curves. Lines from these speeds are dropped vertically to the cumulative frequency distribution curve. The percentage of vehicles traveling at or below each of these speeds can then be determined from the vertical axis of the cumulative frequency distribution curve, as shown. Then:

Even though speeds between 34 and 62 mi/h were observed in this study, over 70% of the vehicles traveled at speeds between 43.5 and 53.5 mi/h. This represents normal traffic behaviour with a standard deviation of approximately 5 mi/h.

Before and After Spot Speed Studies

There are many traffic engineering actions that can help reduce speeds, including lowered speed limits, stricter enforcement measures, warning signs. The major study issue, however, is to demonstrate that speeds have indeed been successfully reduced.

The statistical tests for the significance of observed differences have four possible results:

(1) the actual difference is significant, and the statistical test determines that it is significant;

(2) the actual difference is not significant, and the statistical test determines that it is not significant;

(3) the actual difference is significant and the statistical test determines that it is not significant;

(4) the actual difference is not significant and the statistical test determines that it is significant.

The normal approximation is applied by converting the observed reduction in mean speeds to a value of “z” on the standard normal distribution:

If the target speed lies within this range, it can be stated that it was successfully achieved.

Testing for Normalcy: The Chi-square Goodness-of- Fit Test

Virtually all of the statistical analyses of this section start with the basic assumption that the speed distribution can be mathematically represented as normal. For completeness, it is therefore necessary to conduct a statistical test to confirm that this assumption is correct.

Table 3. Chi-square Test for Normalcy on Illustrative Spot Speed Data.

Proper Location for Speed Studies

The location at which speed measurements are taken must conform to the intended purpose of the study. If approach speeds at a toll plaza are deemed to be too high, measurements should be taken at a point before drivers start to decelerate. Similarly, if excessive speed around a curve is thought to be contributing to off-the-road accidents, speed measurements should be taken in advance of the curve, before deceleration begins.

It may also be appropriate, however, to measure speeds at the point where accidents are occurring for comparison with approach speeds. This would allow the traffic engineer to assess whether the problem is excessive approach speed or that drivers are not decelerating sufficiently through the subject geometric element, or a combination of both. A study of intersection approach speeds must also be taken at a point before drivers begin to decelerate. This may be a moving point, given that queues get shorter and longer at different periods of the day.

Example

Table (2) shows the data collected on a rural highway in Virginia during a speed study. Develop the frequency histogram and the frequency distribution of the data and determine:

1. The arithmetic mean speed

2. The standard deviation

3. The median speed

4. The pace

5. The mode or modal speed

6. The 85th-percentile speed

Table (2) : Speed Data Obtained on Rural Highway.

Solution:

The speeds range from 34.8 to 65.0 mi/h, giving a speed range of 30.2. For eight classes, the range per class is 3.75 mi/h; for 20 classes, the range per class is 1.51 mi/h. It is convenient to choose a range of 2 mi/h per class which will give 16 classes. A frequency distribution table can then be prepared, as shown in Table (3).

in which the speed classes are listed in column 1 and the mid values are in column 2. The number of observations for each class is listed in column 3; the cumulative percentages of all observations are listed in column 6.

Table (3): Frequency Distribution Table for Set of Speed Data.

Fig.3  shows the frequency histogram for the data shown in Table (3). The values in columns 2 and 3 of Table (3)are used to draw the frequency histogram, where the abscissa represents the speeds and the ordinate the observed frequency in each class.

Fig.4  shows the frequency distribution curve for the data given. In this case, a curve showing percentage of observations against speed is drawn by plotting values from column 5 of Table (3) against the corresponding values in column 2. The total area under this curve is one or 100 percent.

Fig.5  shows the cumulative frequency distribution curve for the data given. In this case, the cumulative percentages in column 6 of Table (3)are plotted against the upper limit of each corresponding speed class. This curve, therefore, gives the percentage of vehicles that are traveling at or below a given speed.

Fig.3 Histogram of Observed Vehicle Speed.

Fig.4 Frequency Distribution.

Fig.5 Cumulative Distribution.

The median speed is obtained from the cumulative frequency distribution curve (Fig. 5) as 49 mi/h, the 50th-percentile speed.

• The pace is obtained from the frequency distribution curve (Fig. 4) as 45 to 55 mi/h.

• The mode or modal speed is obtained from the frequency histogram as 49 mi/h (Fig. 3). It also may be obtained from the frequency distribution curve shown in Fig. 4, where the speed corresponding to the highest point on the curve is taken as an estimate of the modal speed.

• 85th-percentile speed is obtained from the cumulative frequency distribution curve as 54 mi/h (Fig. 5).

Example

Solution: