Cloud Physics Lab

LAB 12: Weather Radar

Purpose:

Study the basics of weather radar

Theory:

- Many scattering elements simultaneously illuminated by a transmitted pulse.

- Raindrops, snowflakes, and cloud droplets.

- Resolution volume: Volume containing those particles that are simultaneously illuminated. It is Function of beam width and pulse length.

- Targets move relative to each other.

- Power returned fluctuates in time. The instantaneous power depends on the arrangement of scatters and their motion.

The weather radar equation for distributed targets is given by:

Pr=Pt G2 θ2 π3cτ1024ln2λ2K2R2 Z                                                      (1)

where

Pr= the average received power [watt]

Pt = peak transmitted power [watt]

G = antenna gain [dimensionless]

θ  = beam width [deg]

c = speed of light = 3 × 108 m/s

τ = pulse duration [sec]

λ = radar wavelength [m]

R = range to target [m]

Z = radar reflectivity [m6/m3]

K = complex refractive term of target [dimensionless]

Typical values of radar parameters

| Radar parameter | Typical value |
| --- | --- |
| Pt (peak transmitted power) | 10 – 1000 kw |
| G (antenna gain) | 40 dB |
| θ (beam width) | 1 deg |
| τ (pulse duration) | 0.1 – 5 µsec |
| λ (radar wavelength) | 1 – 10 cm |

The radar reflectivity is defined by:

Z=iDi6unitvolume                                                         (2)

So the unit of Z is (mm6 m-3). Conventionally, Z is expressed in decibel units (dBz) as follows:

ZdBz=10log10Z(mm6 m-3)1(mm6 m-3)                                             (3)

K is a function of temperature (T), wavelength (λ), and sphere composition

|K|2  is approximately equal  0.93 for water and 0.21 for ice.

Methodology

- Assume a radar with the following parameters:

| Pt (peak transmitted power) | 250 kw |
| --- | --- |
| Pr( the average received power) | 0.5 × 10-13 w |
| G (antenna gain) | 40 dB |
| θ (beam width) | 1 deg |
| τ (pulse duration) | 1 µsec |

- Use the Matlab script Lab12.m to plot radar reflectivity (in dBz) for range between 10 and 100 km step 10 km.

- Discuss your results