Lec 5                                 Immunological Techniques

Prof.Dr.Ekhlass Noori Ali

Flow Cytometry

Flow cytometry is a system in which single cells (or beads) in a fluid

suspension are analyzed in terms of their intrinsic light-scattering

characteristics. The cells are simultaneously evaluated for their extrinsic

properties (i.e., the presence of specific surface or cytoplasmic proteins)

using fluorescent-labeled antibodies or probes. Flow cytometers, originally

developed in the 1960s, did not make their way into the clinical laboratory

until the early 1980s. At that point, physicians started seeing patients with a

new mysterious disease characterized by a decrease in circulating T helper

(Th) cells. Since that time, flow cytometry has been routinely used for

monitoring HIV infection status, as well as immunophenotyping cells, or

identifying their surface and cytoplasmic antigen expression.

Flow cytometers can simultaneously measure multiple cellular or bead

properties by using several different fluorochromes. A fluorochrome, or

fluorescent molecule, is one that absorbs light across a spectrum of

wavelengths and emits light of lower energy across a spectrum of longer

wavelengths. Each fluorochrome has a distinctive spectral pattern of

absorption (excitation) and emission. By using laser light, different

populations of cells or particles can be analyzed and identified on the basis of

their size, shape, and antigenic properties.

Flow cytometry is frequently used in leukemia and lymphoma

characterizations as well as in the identification of immunodeficiency

diseases such as AIDS . Flow cytometry is also used to enumerate hematopoietic stem cells, detect human leukocyte antigen (HLA) antibodies in transplantation, and identify cells undergoing apoptosis.

In addition, flow cytometry has been applied in functional assays for chronic

granulomatous disease (CGD) and leukocyte adhesion deficiency, fetal red

blood cell (RBC) and F-cell identification in maternal blood, and

identification of paroxysmal nocturnal hemoglobinuria (PNH), to give just a

few examples. A significant advantage of flow cytometry is that because the

flow rate of cells within the cytometer is so rapid, thousands of events can be

analyzed in seconds, allowing for the accurate detection of cells that are

present in very small numbers.

Instrumentation

The major components of a flow cytometer include the fluidics, the laser

light source, and the optics and photodetectors. Data analysis and management are performed by computers.

Fluidics

For cellular parameters to be accurately measured in the flow cytometer, it is

crucial that cells pass through the laser light one cell at a time. Cells are

processed into a suspension; the cytometer draws up the cell suspension and

injects the sample inside a carrier stream of isotonic saline (sheath fluid) to

form a laminar flow. The sample stream is constrained by the carrier stream

and is thus hydrodynamically focused so that the cells pass single file

through the intersection of the laser light source (Fig. 1). Each time a cell

passes in front of a laser beam, light is scattered, and the interruption of the

laser signal is recorded.

Laser Light Source

Solid-state diode lasers are typically used as light sources. The wavelength of

monochromatic light emitted by a laser in turn dictates which fluorochromes

can be used in an assay. Not all fluorochromes can be used with all lasers

because each fluorochrome has distinct spectral characteristics. Newer

instruments have up to five lasers—red, blue, violet, ultraviolet (UV), and

yellow-green—each of which produces different colors when exciting a

particular fluorochrome. This allows for as many as 20 fluorochromes, or

colors, to be analyzed in a single tube at one time.

Because of a cell passing through the laser, light is scattered in many

directions. The amount and type of light scatter (LS) can provide valuable

information about a cell’s physical properties. Light at two specific angles is

measured by the flow cytometer: (1) forward scatter (FSC) and (2) side

scatter (SSC), also called right-angle LS. FSC is considered an indicator of

size, whereas the SSC signal is indicative of granularity or the intracellular

complexity of the cell. Thus, these two values can be used to characterize

different cell types based on their inherent properties and are considered

intrinsic parameters. If one looks at a sample of whole blood on a flow

cytometer where all the RBCs have been lysed, the three major populations

of white blood cells (WBCs)—lymphocytes, monocytes, and neutrophils—

can be roughly differentiated from each other based solely on their intrinsic

parameters (FSC and SSC) (Fig. 2).

Unlike FSC and SSC, which represent light-scattering properties that are

intrinsic to the cell, extrinsic parameters require the addition of a fluorescent probe for their detection. Fluorescent-labeled antibodies bound to

the cell can be detected with the laser. By using fluorescent molecules with

various emission wavelengths, laboratory scientists can simultaneously

evaluate an individual cell for several extrinsic properties. The clinical utility

of such multicolor analysis is enhanced when the fluorescent data are

analyzed in conjunction with FSC and SSC to enable more accurate

subtyping. The combination of data allows for characterization of cells

according to size, granularity, DNA and RNA content, antigens, total protein,

and cell surface receptors.

FIGURE 1 Flow cytometry. Components of a laser-based flow cytometer include the

fluidics system for cell transportation, a laser for cell illumination, photodetectors for signal

detection, and a computer-based data management system. Both forward and 90-degree LS

are measured, indicating cell size and type.

FIGURE 2 Peripheral blood leukocyte analysis by simultaneous evaluation of forward light

scatter (FSC) and 90-degree LS (SSC). Based on the intrinsic characteristics of size (FSC)

and complexity/granularity (SSC), the three main populations of WBCs (lymphocytes,

monocytes, and neutrophils) can be discriminated into individual populations.

Optics and Photodetectors

The various signals (light scatter and fluorescence) generated by the cells’

interaction with the laser are detected by photodiodes for FSC and by

photomultiplier tubes for fluorescence. The number of fluorochromes capable

of being measured simultaneously depends upon the number of

photomultiplier tubes in the flow cytometer. The specificity of each

photomultiplier tube for a given band length of wavelengths is achieved

through the arrangement of a series of optical filters that are designed to

maximize collection of light derived from a specific fluorochrome while

minimizing interference of light from other fluorochromes. The newer flow

cytometers use fiber-optic cables to direct light to the detectors.

When fluorescence from labeled antibodies bound to cell surfaces reaches

the photomultiplier tubes, it creates an electrical current that is converted into

a voltage pulse. The voltage pulse is then converted into a digital signal using

various methods, depending on the manufacturer. The digital signals are

proportional to the intensity of light detected. The intensity of these

converted signals is measured on a relative scale that is generally set into 1 to

256 channels, from the lowest energy level or pulse to the highest level.

Sample Preparation

Samples commonly used for analysis include whole blood, bone marrow, and

fluid aspirates. Whole blood should be collected into

ethylenediaminetetraacetic acid (EDTA), the anticoagulant of choice for

samples processed within 30 hours of collection. Heparin can also be used for

whole blood and bone marrow and can provide improved stability in samples

for up to 48 hours. Blood should be stored at room temperature (20°C to

25°C) before processing and should be well mixed before being pipetted into

staining tubes. Hemolyzed or clotted specimens should be rejected.

Peripheral blood, bone marrow, and other samples with large numbers of

RBCs require erythrocyte removal to allow for efficient analysis of WBCs.

Historically, density gradient centrifugation with Ficoll-Hypaque (Sigma, St.

Louis, MO) was used to generate a cell suspension enriched for lymphocytes

or lymphoblasts. However, this method results in selective loss of some cell

populations and is time-consuming. Density-gradient centrifugation has

mainly been replaced by erythrocyte lysis techniques, both commercial and

non-commercial. Samples are treated with lysing buffers to destroy the

erythrocytes while leaving the WBCs intact.

Tissue specimens such as lymph nodes should be collected and transported

in tissue culture medium (RPMI 1640) at either room temperature (if analysis

is imminent) or 4°C (if analysis will be delayed). The specimen is then

disaggregated to form a single cell suspension, either by mechanical

dissociation or enzymatic digestion. Mechanical disaggregation, or “teasing,”

is preferred and is accomplished by the use of a scalpel and forceps, a needle

and syringe, or a wire mesh screen. Antibodies are then added to the resulting

cellular preparation and the samples are processed for analysis. The antibodies used are typically monoclonal, each labeled with a different fluorescent tag.

Data Acquisition and Analysis

Once the intrinsic and extrinsic properties of the cells have been collected,

the data are digitized and ready for analysis. Typically 10,000 to 20,000

“events” are collected for each sample. Each parameter can be analyzed

independently or in any combination. Graphics of the data can be represented

in multiple ways. The first level of representation is the single-parameter

histogram, which plots a chosen parameter (generally fluorescence) on the

x-axis versus the number of events on the y-axis; thus, only a single

parameter is analyzed using this type of graph (Fig. 3). The operator can

then set a marker to differentiate cells that have low levels of fluorescence

(negative) from cells that have high levels of fluorescence (positive) for a

particular fluorochrome-labeled antibody. The computer will then calculate

the percentage of “negative” and “positive” events from the total number of

events collected.

The next level of representation is the bivariate histogram, or dualparameter

dot plot, where each dot represents an individual cell or event.

Two parameters, one on each axis, are plotted against each other. Each

parameter to be analyzed is determined by the operator. Using dual-parameter

dot plots, the operator can draw a “gate” around a population of interest and

analyze various extrinsic and intrinsic parameters of the cells contained

within the gated region (Fig. 4). The gate allows the operator to screen

out debris and isolate subpopulations of cells of interest. Gates can be

thought of as a set of filtering rules for analyzing a very large database. The

operator can filter the data in any way and set multiple or sequential filters

(or gates).

FIGURE 3 Example of a single-parameter flow histogram. The y-axis consists of the number of events. The x-axis is the parameter to be analyzed, which is chosen by the operator; it is usually an extrinsic parameter, such as a fluorescence. The operator can then set a marker to isolate the positive events. The computer will then calculate the percentage of positive events within the designated markers.

FIGURE 4 A dual-parameter dot plot. Both parameters on the x- and y-axes are chosen by

the operator. In this case, lysed whole blood is analyzed for CD45 (x-axis) and SSC (y-axis).

The operator then draws a “gate” to isolate the population of interest (e.g., granulocytes) for

further analysis.

When analyzing a population of cells using a dual-parameter dot plot, the

operator chooses which parameters to analyze on both the x- and y-axes,

divides the dot plot into four quadrants, and separates the positive events

from the negative events in each axis (Fig. 5). Quadrant 1 consists of

cells that are positive for fluorescence on the y-axis and negative for

fluorescence on the x-axis. Quadrant 2 consists of cells that are positive for

fluorescence on both the x- and y-axes. Quadrant 3 consists of cells that are

negative for fluorescence on both the x- and y-axes. Quadrant 4 consists of

cells that are positive for fluorescence on the x-axis and negative for

fluorescence on the y-axis. The computer and specialized software will

calculate the percentage of cells in each quadrant based on the total number

of events counted.

FIGURE 5 Quadrant analysis of a dual-parameter dot plot. The operator chooses which

parameters to analyze on each axis. (A) On each axis, there are fluorescence positive and

fluorescence negative cells. (B) Example of a dual-parameter dot plot to identify CD4+ T cells:

CD3 on the x-axis and CD4 on the y-axis. The cells in quadrant 2 that are positive for both

CD3 and CD4 are true CD4+ T cells.

A gate can be drawn around a population of cells based on their FSC

versus SSC characteristics, and the extrinsic characteristics of the gated

population can then be analyzed. For example, lymphocytes can be gated,

after which the T-cell subpopulations (CD3+, CD4+ or CD3+, CD8+) and B

cells (CD38+, CD3–) can be analyzed (Fig. 6). The absolute count of a

particular cell type—for instance, CD4+ T lymphocytes—can be obtained by

multiplying the absolute cell count of the population of interest (e.g.,

lymphocytes) derived by a hematology analyzer by the percentage of the

fluorescent-positive cells in the sample CD3+, CD4+ lymphocytes. This

method is considered a dual-platform analysis. The disadvantage to this type

of analysis is that it has a greater potential for added error associated with the

use of two distinct methods to derive the absolute count. The single platform

is now the method of choice to eliminate this type of error. Single platforms

can be achieved by two types of methods: bead-based or volumetric. In the

bead-based method, a known quantity of fluorescent beads is added to the

flow cytometry tubes, and a simple mathematic calculation allows the

absolute WBC numbers to be directly determined from the individual flow

cytometry tubes. In the volumetric method, the precise volume of the sample

can be used to calculate the absolute number of events.

Detailed phenotypic analysis can determine the lineage and clonality, as

well as the degree of differentiation and activation, of a specific cell

population. This information is useful for differential diagnosis or

clarification of closely related lymphoproliferative disorders.

Immunophenotyping requires careful selection of combinations of

individual markers based on a given cell lineage and maturation stage.

Attempts to standardize individual marker panels, especially by European

laboratory groups, are ongoing; however, the markers selected for inclusion

in testing panels vary from institution to institution.

Clinical Applications

Routine applications of flow cytometry in the clinical laboratory can be

divided into two categories: nonmalignant immunophenotyping and

malignant immunophenotyping. Nonmalignant immunophenotyping includes

the characterization and enumeration of normal lymphocytes, such as CD4+

T cells and CD34+ stem cells. Malignant immunophenotyping refers to

immunophenotypic characterization of leukemias, lymphomas, and other

hematopoietic disorders.

Immunophenotyping by flow cytometry has become an important

component of initial evaluation and subsequent post-therapeutic monitoring

in leukemia and lymphoma management. Flow cytometric findings have

been incorporated into current leukemia and lymphoma classifications,

beginning with the Revised European-American Lymphoma (REAL)

classification in 1994 and, more recently, in the World Health Organization

(WHO) classifications. One of the most important components of flow

cytometric analysis is the stratification of hematopoietic malignancies by

their lineage (i.e., B cell, T cell, or myeloid) and degree of differentiation.

Some of the more common cell-differentiation antigens are listed in Table

13–1.

FIGURE 6 Gating strategy to analyze lymphocyte subsets in a sample of whole blood.

Whole blood is incubated with fluorescent-labeled antibodies specific for CD3, CD4, CD8, and

HLA-DR. The sample is washed, RBCs are lysed, and the sample is analyzed on the flow

cytometer. To analyze using gating strategies, the sample is first plotted using FSC versus

SSC. (A) A gate, or region, is drawn around the lymphocyte population. (B) On the

subsequent plots of fluorescent markers, only the lymphocyte population is analyzed. The dot

plot is divided into four quadrants to isolate positive from negative populations. The computer

calculates the percentage of positive cells in each quadrant. The three flow contour plots are

analyzing two different cell surface markers. In the first dot plot, quadrant 2 (upper right)

identifies CD4+ CD3+ T-helper-cell lymphocytes. Quadrant 3 (lower left) includes B

lymphocytes and natural killer (NK) cells. Quadrant 4 (lower right) identifies CD3+ CD4– T-cell

lymphocytes. In the second dot plot, quadrant 1 (upper left) identifies low-intensity CD8+

CD3– NK cells. Quadrant 2 identifies CD3+ CD8+ T-cytotoxic lymphocytes. Quadrant 3

contains any lymphocyte that is not CD8+, CD3+ (B-cell lymphocytes). Quadrant 4 contains

CD3+ CD8– T helper cell lymphocytes. In the third contour plot, quadrant 1 identifies CD38+

CD3– B cells, quadrant 2 identifies CD38+ CD3+ activated T cells, quadrant 3 contains

CD38– CD3– cells, and quadrant 4 identifies CD3+ CD38– T cells, not activated.

Knowing the unique characteristics of leukemias and lymphomas and

pairing the specific markers that identify these characteristics can be useful in

making a more reliable diagnosis. For example, in chronic lymphocytic

leukemia (CLL), typically CD5 (a T-cell lymphocyte marker) is paired with

CD20 (a B-cell lymphocyte marker) for analysis. The presence of a

significant number of cells that are both CD5+ and CD20+ is an indication of

CLL or mantle cell lymphoma. Common marker combinations are shown in

Table 13–2.

CD45 is a pan-leukocyte marker present on all WBCs but with varying

levels of expression based on the cell’s maturity as well as lineage. This

variance in expression results in different levels of fluorescence. Blasts

express lower levels of CD45 (low fluorescence) but show an increase of

CD45 expression as the cell matures; therefore, mature WBCs have much

brighter CD45 fluorescence compared with their earlier progenitor stages.

The analysis of CD45 expression levels is useful in differentiating various

populations of WBCs and, in combination with SSC, has replaced gating by

FSC and SSC in many laboratories. However, it is always a good idea to

examine the FSC and SSC plot to be sure a population is not being missed.

Immunophenotyping of WBC populations is also essential when an

immunodeficiency is suspected. Patient values are compared with reference

ranges established by the clinical laboratory performing the testing. Typical

ranges for B cells, T-cell subsets, and natural killer (NK) cells are listed in

Table 13–3.

Flow cytometry is essential to the evaluation of immunodeficiency

resulting from infection with HIV, which can lead to AIDS. Enumeration of

peripheral blood CD4+ T cells in HIV-infected patients by the flow

cytometry laboratory is used to classify stages of HIV disease and determine

treatment options. HIV type 1 (HIV-1) infections cause a rapid, profound decrease in CD4+ T-cell numbers and an expansion of CD8+T-cell levels during the early course of the illness (12 to 18 months). Some individuals continue to rapidly lose CD4+ T cells and progress to AIDS,whereas others maintain relatively stable CD4+ T-cell counts and remain AIDS-free for years. During the chronic phase of HIV-1 disease, there can be a slow decline in CD4+ T cells throughout many years because of available therapies and maintenance of homeostatic mechanisms in the patient. If these mechanisms fail, further declines in CD4+ T and CD8+ T cells occur, leading to the development of AIDS.

Flow cytometry is also the method of choice for the diagnosis of several

inherited immunodeficiency diseases such as Bruton’s tyrosine kinase

deficiency, which is characterized by a lack of circulating mature B cells

(CD19+). Another example is chronic granulomatous disease, an X-linked or

autosomal recessive disease in which neutrophils are defective in their

oxidative burst. In this case, neutrophils are exposed to the dye

dihydrorhodamine 123, which is not fluorescent until oxidized. When normal

neutrophils are stimulated in vitro, the dye is oxidized and becomes intensely

fluorescent. Neutrophils from patients with CGD are unable to oxidize the

dye and do not become fluorescent after stimulation.

Another example is paroxysmal nocturnal hemoglobinuria (PNH), an

inherited disease characterized by hemolytic anemia. The RBCs, neutrophils,

monocytes, and other cells of patients with PNH lack the

glycosylphosphatidylinositol (GPI) anchors by which many surface proteins

are attached to the cell membrane. Therefore, the RBCs are fragile.

Hemolysis occurs during pH changes in the blood, typically at night. Several

flow cytometry tests are available to detect this defect. One test looks for the

antigens that use the GPI anchor because these antigens will be missing or

reduced in patients with PNH. Another test detects the GPI anchor itself by

its ability to bind to fluorescent aerolysin (FLAER). Normal granulocytes and

monocytes will be fluorescent because they possess the anchor, whereas the

fluorescence in PNH cells missing the anchor will be reduced or absent.

Immunophenotyping has also become an essential test in the diagnosis and

monitoring of patients with leukemias and lymphomas .

Finding a small number of abnormal cells in a particular cell population can

be easily accomplished by flow cytometry. Patients who have been treated

for leukemia or lymphoma can be monitored for “minimal residual disease”

(MRD) because statistically significant rare cell events can be easily

detected. Likewise, in the case of a fetal-maternal hemorrhage, using flow

cytometry to detect hemoglobin F-positive cells is much more sensitive than

the traditional Kleihauer–Betke method.

Additional examples of flow cytometry use include the determination of

DNA content or ploidy status of tumor cells. This analysis can provide

physicians with important prognostic information. Ploidy analysis is also

useful in examining products of conception for molar pregnancies.

Flow cytometry also has applications in transplant immunology. HLA

typing and cross-matching for solid-organ transplantation can be performed

by flow cytometry much faster and more accurately than other serological

methods .

Finally, other important uses for flow technology in clinical diagnosis

involve cytometric bead arrays. In this technology, beads of various sizes and

different fluorescent levels are used to identify multiple analytes at the same

time. Theoretically, this technology has the potential to detect 100 analytes

from a single blood sample.

For example, in testing for anti-nuclear antibodies, different colored beads

can be coated with different nuclear antigens. Patient serum is then added,

followed by fluorescent anti-IgG. If the patient has a particular anti-nuclear

antibody, the respective bead will fluoresce and can be identified and

quantitated . Similarly, molecular techniques such as polymerase chain reaction (PCR) and hybridization can be performed on cytometric beads to detect viral nucleic acids and various genetic mutations

Flow cytometry relies on the use of fluorescent-labeled monoclonal and

polyclonal antibodies. Monoclonal antibodies are preferable to polyclonal

antibodies for immunophenotyping. The commercial availability of monoclonal antibody reagents has contributed greatly to the accuracy of flow cytometry and has widened its use.

Connections